Open Geospatial Consortium

Submission Date: 2016-31-08

Approval Date:   <yyyy-dd-mm>

Publication Date:   <yyyy-dd-mm>

External identifier of this OGC® document: <http://www.opengis.net/doc/[{doc-type/}]{standard}/{m.n}>

Internal reference number of this OGC® document:    14-111r2

Version: 0.6

Category: OGC® Implementation Standard

Editors:    David Blodgett, Irina Dornblut

OGC®  WaterML 2 – Part 3:  Surface Hydrology Features (HY_Features) – Conceptual Model

Copyright notice

Copyright © 2016 Open Geospatial Consortium
To obtain additional rights of use, visit http://www.opengeospatial.org/legal/.

Warning

This document is not an OGC Standard. This document is distributed for review and

comment. This document is subject to change without notice and may not be referred to

as an OGC Standard.

Recipients of this document are invited to submit, with their comments, notification of

any relevant patent rights of which they are aware and to provide supporting

documentation.

Document type:   	OGC® Implementation Standard

Document subtype:

Document stage:   	Draft

Document language: 	English

License Agreement

Permission is hereby granted by the Open Geospatial Consortium, ("Licensor"), free of charge and subject to the terms set forth below, to any person obtaining a copy of this Intellectual Property and any associated documentation, to deal in the Intellectual Property without restriction (except as set forth below), including without limitation the rights to implement, use, copy, modify, merge, publish, distribute, and/or sublicense copies of the Intellectual Property, and to permit persons to whom the Intellectual Property is furnished to do so, provided that all copyright notices on the intellectual property are retained intact and that each person to whom the Intellectual Property is furnished agrees to the terms of this Agreement.

If you modify the Intellectual Property, all copies of the modified Intellectual Property must include, in addition to the above copyright notice, a notice that the Intellectual Property includes modifications that have not been approved or adopted by LICENSOR.

THIS LICENSE IS A COPYRIGHT LICENSE ONLY, AND DOES NOT CONVEY ANY RIGHTS UNDER ANY PATENTS THAT MAY BE IN FORCE ANYWHERE IN THE WORLD.

THE INTELLECTUAL PROPERTY IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, AND NONINFRINGEMENT OF THIRD PARTY RIGHTS. THE COPYRIGHT HOLDER OR HOLDERS INCLUDED IN THIS NOTICE DO NOT WARRANT THAT THE FUNCTIONS CONTAINED IN THE INTELLECTUAL PROPERTY WILL MEET YOUR REQUIREMENTS OR THAT THE OPERATION OF THE INTELLECTUAL PROPERTY WILL BE UNINTERRUPTED OR ERROR FREE. ANY USE OF THE INTELLECTUAL PROPERTY SHALL BE MADE ENTIRELY AT THE USER’S OWN RISK. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR ANY CONTRIBUTOR OF INTELLECTUAL PROPERTY RIGHTS TO THE INTELLECTUAL PROPERTY BE LIABLE FOR ANY CLAIM, OR ANY DIRECT, SPECIAL, INDIRECT OR CONSEQUENTIAL DAMAGES, OR ANY DAMAGES WHATSOEVER RESULTING FROM ANY ALLEGED INFRINGEMENT OR ANY LOSS OF USE, DATA OR PROFITS, WHETHER IN AN ACTION OF CONTRACT, NEGLIGENCE OR UNDER ANY OTHER LEGAL THEORY, ARISING OUT OF OR IN CONNECTION WITH THE IMPLEMENTATION, USE, COMMERCIALIZATION OR PERFORMANCE OF THIS INTELLECTUAL PROPERTY.

This license is effective until terminated. You may terminate it at any time by destroying the Intellectual Property together with all copies in any form. The license will also terminate if you fail to comply with any term or condition of this Agreement. Except as provided in the following sentence, no such termination of this license shall require the termination of any third party end-user sublicense to the Intellectual Property which is in force as of the date of notice of such termination. In addition, should the Intellectual Property, or the operation of the Intellectual Property, infringe, or in LICENSOR’s sole opinion be likely to infringe, any patent, copyright, trademark or other right of a third party, you agree that LICENSOR, in its sole discretion, may terminate this license without any compensation or liability to you, your licensees or any other party. You agree upon termination of any kind to destroy or cause to be destroyed the Intellectual Property together with all copies in any form, whether held by you or by any third party.

Except as contained in this notice, the name of LICENSOR or of any other holder of a copyright in all or part of the Intellectual Property shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Intellectual Property without prior written authorization of LICENSOR or such copyright holder. LICENSOR is and shall at all times be the sole entity that may authorize you or any third party to use certification marks, trademarks or other special designations to indicate compliance with any LICENSOR standards or specifications. This Agreement is governed by the laws of the Commonwealth of Massachusetts. The application to this Agreement of the United Nations Convention on Contracts for the International Sale of Goods is hereby expressly excluded. In the event any provision of this Agreement shall be deemed unenforceable, void or invalid, such provision shall be modified so as to make it valid and enforceable, and as so modified the entire Agreement shall remain in full force and effect. No decision, action or inaction by LICENSOR shall be construed to be a waiver of any rights or remedies available to it.

Contents

1.	Scope	12

2.	Conformance	15

3.	Normative References	16

4.	Terms and Definitions	16

5.	Conventions	25

5.1	Identifiers	25

5.2	Symbols (and abbreviated terms)	25

5.3	UML notation	26

5.4	WMO Terminology	26

5.5	Naming convention	27

6.	Clauses not Containing Normative Material	27

6.1	The abstract idea of the hydrology phenomenon	27

6.2	Catchment and realizations of the catchment concept	30

6.3	Catchment hierarchy and network topology	33

6.3.1	Hierarchy of catchments	34

6.3.2	Topological network of catchments	36

6.4	River reference system	41

6.5	Hydrographic and hydrometric networks	44

6.5.1	Hydrographic and channel networks	44

6.5.2	Hydrometric networks	45

6.5.3	HydroNetwork: hydrographic network, channel network and station network - different views of catchment topology	46

6.5.4	FlowPath	47

6.6	Discussion of purpose and relation to ISO standards baselines.	48

6.6.1	Catchment topology, comparison with the ISO topology model	48

6.6.2	River Reference System, comparison with ISO linear referencing model	49

6.6.3	Catchment Network Navigation, comparison with ISO network (navigation) model	51

7.	Clause containing normative material	52

7.1	The HY_Features conceptual model	52

7.2	The HY_Features conceptual conformance (mapping)	53

7.3	The HY_Features data conformance (encoding)	54

7.4	The Hydro Feature application schema	55

7.4.1	The Named Feature model	57

7.4.2	The Hydro Complex model	59

7.4.3	The River Positioning System model	78

7.5	The Surface Hydro Feature application schema	81

7.5.1	The Channel Network model	83

7.5.2	The Hydrographic Network model	88

7.5.3	The Surface-Water Body types	96

7.5.4	The Storage model	96

7.6	The Hydrometric Network application schema	98

ANNEX A Conformance Class Abstract Test Suite (Normative)	1

A.1 Introduction	1

A.2 Conformance class: HY_HydroFeature application schema equivalence	1

A.3 Conformance class: HY_SurfaceHydroFeature application schema equivalence	2

A.4 Conformance class: HY_HydrometricFeature application schema equivalence	2

ANNEX B - Code lists for the HY_Features model	1

B.1 Terms identifying a fixed landmark determined to realize the conceptual outfall	1

B.2 Terms commonly used in hydrology to describe a spatial relation between two points	2

B.3 Terms commonly used in hydrology to describe a drainage pattern	2

B.4 Terms commonly used to indicate the type of name usage.	3

ANNEX C:  HY_Features  - AHGF Mapping	1

C.1 Catchment Model	1

C.2 Hydrographic Network	2

C.3 Hydrometric Network	4

ANNEX D:  HY_Features  - NHDPlus  Mapping	1

D.1 Catchment Model	1

D.2 Hydrographic Network Model	2

D.3 Hydrometric Network Model	3

ANNEX E:  HY_Features  - INSPIRE Hydrography Theme	1

E.1 Catchment Model	1

E.2 Hydrographic Network Model	3

ANNEX F:  HY_Features  - SANDRE  Mapping	1

F.1 Catchment Model	1

F.2 Hydrographic Network Model	2

F.3 Hydrometric Network Model	3

ANNEX G: Bibliography	1

ANNEX H: Revision history	1

Figures

Figure 1: HY_Features in the context of the OGC Abstract Specifications	14

Figure 2: Processes of the Hydrologic Cycle	28

Figure 3: Illustration of multiple representations of a catchment	30

Figure 4: Idealized catchment. This idealized catchment diagram is used to represent the catchment concept with no emphasis on any realization or representation. In the diagrams below, aspects of this diagram are highlighted to emphasize particular concepts of the HY_Features conceptual model.	31

Figure 5: Multiple graphical representations of a catchment (from top left to bottom right): a) Catchment boundary, b)   Catchment area, c) flowpath of catchment d) network of sub catchments, e) cartographic view, f) abstract flow paths, g) hydrographic network, h) network of logically connected monitoring stations.	32

Figure 6 (C1-C5, from left to right): C1, Typical catchments with one inflow and one outflow each; C2, Joined (conjoint) catchments flowing into a single downstream catchment; C3, catchments joining in a water body or wetland with no clear network; C4, catchments joining through intermittent or subsurface flows C5, catchments that join through areas of complex or braided channels.	34

Figure 7: Catchment hierarchy – A catchment (dark grey), may be nested within a containing catchment which is another catchment (light grey)	35

Figure 8: Catchment hierarchy – A catchment (C1, C2, or C3) may be part of a simple dendritic network of catchments which is also a catchment.	36

Figure 9: A catchment area (grey) and a flowpath connecting inflow to outflow (red) depicting a definable unit where hydrological processes take place.	37

Figure 10: Catchments contributing to an identified outflow node. Note that some catchments contribute to a common outflow node.	38

Figure 11: Catchments receive inflow via an identified inflow node. Note that nodes are not necessarily geographic features, but are rather nodes in a graph representation of the river network.	39

Figure 12: Encapsulation of non-dendritic stream network topology. The left figure shows a case where it is not possible to determine to what extent flow from catchment F contributes to catchments E, B or C. The right figure shows how catchments E, B, and C can be aggregated so nodes N2 and N3 are treated as a single virtual inflow node, so that all the flow from catchments D and F accumulate in the resulting catchment X.	40

Figure 13: Position (yellow dot) downstream of a reference point (red dot). While referenced positions (P) are usually referenced to permanent locations like confluences (r2), they can also be considered to be the outflow of contributing catchment and thus, the origin of a river reference system of their own.	42

Figure 14: Position (yellow dot) upstream of a reference point (red dot). Typically, r1 would be the origin and P, the referenced point; however, the river reference system model allows for the referenced point to be the origin of its own river reference system.	43

Figure 15: NEW  Position (yellow dot) between two a reference points (red dots). 	44

Figure 16: Cyclic nature of the HY_Features basic model	47

Figure 17: HY_Features modules and packages	53

Figure 18: Hydrologic features describing separate aspects of the hydrology phenomenon	56

Figure 19: Hydro Feature – external dependencies	57

Figure 20: Named Feature (UML class diagram, /req/hy_hydrofeature/namedFeature/hydrofeature )	58

Figure 21: Catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment )	60

Figure 22: Catchment and outfall (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.inflow , /req/hy_hydrofeature/hydrocomplex/catchment.outflow )	61

Figure 23: Containing / contained catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.containingcatchment , /req/hy_hydrofeature/hydrocomplex/catchment.containedcatchment )	62

Figure 24: Conjoint catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.conjointcatchment )	62

Figure 25: Upper / lower catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.uppercatchment, /req/hy_hydrofeature/hydrocomplex/catchment.lowercatchment )	62

Figure 26: Catchment aggregate (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchmentaggregate )	64

Figure 27: Dendritic catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/dendriticcatchment )	66

Figure 28: Interior catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/interiorcatchment )	67

Figure 29: Outfall (UML class diagram, req/hy_hydrofeature/hydrocomplex/outfall )	69

Figure 30: Catchment realization and its specialization (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchmentrealisation)	71

Figure 31: Outfall Realization (UML class diagram, /req/hy_hydrofeature/hydrocomplex/outfallrealization )	77

Figure 32: River Positioning System (UML class diagram, /req/hy_hydrofeature/positioning/indirectposition, /req/hy_positioning/riverreferencesystem )	79

Figure 33: Surface Hydro Feature -  dependencies/req/hy_surfacehydrofeature/storage/*	82

Figure 34: Channel Network realizing the catchment (UML class diagram, /req/hy_surfacehydrofeature/channelnetwork/channelnetwork )	83

Figure 35: Depression and Channel realizing the outfall (UML class diagram, /req/hy_surfacehydrofeature/channelnetwork/depression, /req/hy_surfacehydrofeature/channelnetwork/channel )	84

Figure 36: Hydrographic Network realizing the catchment (UML class diagram, /req/hy_surfacehydro/hydrographicnetwork/hydrographicnetwork )	89

Figure 37: Water Body realizing the outfall (UML class diagram, /req/hy_surfacehydro/hydrographicnetwork/waterbody )	90

Figure 38: Reservoir realizing an outfall (UML class diagram, /req/hy_surfacehydrofeature/storage/reservoir )	97

Figure 39: Hydrometric Network – dependencies	98

Figure 40: Hydrometric network model (UML class diagram, /req/hy_hydrometricfeature/hydrometricnetwork )	99

Tables

Table 1: HY_Features modules, concepts reflected, and leaf packages included	46

List of Requirements

Abstract

The OGC Surface Hydrology Features (HY_Features) implementation standard defines a standard information model for the identification of surface hydrologic features independent of geometric representation and scale. The conceptual model describes hydrologic features by defining the fundamental relationships among major components of the hydrosphere. This includes relationships such as the hierarchy of catchments, the segmentation of watercourses, and the topological connectivity of hydrologic features such as catchment and water bodies.

The standard is based on the concept that a given hydrologic feature may have multiple representations. This supports referencing the same feature(s) in different information systems or products assisting the organization on having integrated information system. Moreover, this model also supports the cataloging of observations, model results, or other studies of a feature. The ability to represent the same catchment, river, or other hydrologic feature in several ways is critical to aggregation of cross-referenced features into integrated datasets and data products on global, regional, or basin scales.

Hydrologic feature types are defined using the OGC General Feature Model (ISO 19109:2006) with reference to definitions from the International Glossary for Hydrology. The conceptual model is expressed in the Geographic Information Conceptual Schema Language (ISO 19103:2005) using the Unified Modeling Language (UML).

Keywords

The following are keywords to be used by search engines and document catalogues.

ogcdoc, OGC document, hydrology, feature, identification, conceptual model, UML, implementation standard, hydrology model.

Preface

This standard defines the HY_Features common hydrologic feature model for the identification of hydrologic features. It is intended to be used to document and share information about the objects of study and reporting in Hydrology in many applications. This standard was originally commissioned to link hydrologic information across the scientific and technical programs of the World Meteorological Organization (WMO), and to assist the WMO Members to discover, access and use hydrologic data from different sources.

This standard has been designed to support the need for governance and guidance by national and international authorities. Aspects of the standard that support this end are; 1) its canonical form, 2) its implementation neutrality, 3) conformity to internationally recognized standards of geographic information, and its use of semantics inferred from terminology endorsed by the WMO and the UN Educational, Scientific and Cultural Organization (UNESCO).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. The Open Geospatial Consortium shall not be held responsible for identifying any or all such patent rights.

Recipients of this document are requested to submit, with their comments, notification of any relevant patent claims or other intellectual property rights of which they may be aware that might be infringed by any implementation of the standard set forth in this document, and to provide supporting documentation.

Submitting organizations

- The following organizations contributed to the initiation or development of this standard and submitted this Document to the Open Geospatial Consortium OGC):

U.S. Geological Survey (USGS), USA

Federal Institute of Hydrology (BfG) , Germany

CSIRO Land and Water, Australia

Bureau of Meteorology, Australia

Metalinkage, Australia

French Geological Survey (BRGM), France

University of Texas at Austin, USA

Eurecat, Spain

INCLAM S. A., Spain

52°North, Germany

Submitters

All questions regarding this submission should be directed to the editor or the submitters:

| Name | Affiliation |
| --- | --- |
| David Blodgett | U.S. Geological Survey |
| Irina Dornblut | German Federal Institute of Hydrology (BfG) |
| Robert A. Atkinson | Metalinkage |
| Joshua Liebermann | Harvard University |
| Darren Smith | Australian Bureau of Meteorology |
| Bruce Simons | CSIRO Land and Water |
| David Arctur | University of Texas at Austin |
| Mickael Beaufils | French Geological Survey (BRGM) |
| Aitor Corchero | Eurecat |

Changes to the OGC® Abstract Specification

This standard requires no changes to the OpenGIS ® Abstract Specification.

# Scope

This OGC Implementation Standard defines a common feature model for the identification and description of surface hydrologic features using established models and patterns in use in the Hydrology domain and endorsed by WMO and UNESCO such as those documented in the "International Glossary of Hydrology".

This standard will form the third part of an intended ‘OGC WaterML 2’ suite of standards  which will group the water-related OGC standards. Following Part 1: Timeseries, Part 2: Ratings, Gaugings and Sections, this standard will form Part 3: Surface Hydrology Features.

This document introduces the HY_Features conceptual model itself. The normative model is a machine-readable UML artifact published by the OGC in conjunction with this document at: [insert URL here].

A future document is expected to define a semantic representation (OWL/RDF) of the conceptual model suitable for defining links between data that reference the HY_Features conceptual model. This semantic model is expected to support documentation and discovery of data and to aid future data transformation efforts. This work may provide a basis for further work in the wider OGC on a methodology (tooling) to publish ontologies to support their practical use.

The initial scope is defined by the WMO Commission for Hydrology (WMH-CHy): to facilitate data sharing within the hydrologic community of the WMO Member countries and to improve the quality of data products based on these data by defining hydrologic features to convey the identification of water-objects through the data processing chain "from measurement to hydrological information" [8].

To enable semantic interoperability of hydrologic data and services, it is necessary to agree on common concepts and methods for machine interpretation of the concepts. To this goal, the HY_Features model was developed in order to formalize the concepts and relationships of hydrologic features using the WMO/UNESCO "International Glossary of Hydrology" as a starting point.

This standard is meant to support the linkage of data products distributed across differing applications and jurisdictions. To enable this, a holistic concept of catchment is defined and the surface-water aspects of the concept are modeled. This will allow a particular surface catchment to be represented in different data products while retaining its identity. In addition to unique identification of features in multiple systems, a second objective is to provide a standard terminology and an ontology to describe relationships between hydrologic features. This may be useful in building a data transfer format for specific subsets, particularly catchment hierarchy and river network topology but such data transfer format development is not in scope for this standard.

The HY_Features model provides a basis for common and stable references to hydrologic features in a wide variety of applications:

- to link hydrologic observations to their feature-of-interest, e.g. link a streamflow observation to the river or catchment being observed,

- to allow aggregation of cross-referenced features into integrated datasets and data products on global, regional, or basin scales,

- to enable information systems to unambiguously link data across distributed systems and domains,

- to enable cross-domain or multi-discipline services to communicate through reference to standard concepts.

This standard defines a set of Application Schemas (normative) containing feature type definitions that conform to the OGC General Feature Model (GFM) (ISO 19101:2002 and ISO 19109:2006), expressed in the Geographic Information Conceptual Schema Language (ISO 19103:2005) using the Unified Modeling Language (UML).

The GFM is a meta-model developed to serve as the general conceptual model for features and feature properties in the context of geographic information. A feature type is identified by a set of typical characteristics (property type) such as attributes, association, or operations as well as by possible constraints. Each attribute, association or operation is identified on its own by properties and constraints. Conformity to the GFM is assured by instantiating the general feature type as feature classes specific to the application domain.

As shown in Figure 1, the HY_Features conceptual model realizes the GFM by providing a domain-specific instance of the general GF_FeatureType (aka FeatureType) «metaclass» capturing the Hydrology phenomenon. Since its concern is primarily the issue of feature identification, a basic type HY_HydroFeature is defined to reflect the overall properties hydrologic features have such as identifier and name. Special feature types are defined to reflect different aspects of hydrology by the typical characteristics each specialization carries. Given the complexity of the domain, a wide range of possible properties may be relevant for a given hydrologic feature type.

Figure 1 also shows how the HY_Features model allows the feature-of-interest concept of the Observation and Measurement (O&M) model (ISO 19156:2011) to be realized for hydrologic observations. Depending on the application, the target of an observation may be a Domain Feature, like a water body, or a Sampling Feature, such as a stream gage, used as a proxy for a Domain Feature. Specifically, observation-centric data models such as the WaterML2.0 implementation profiles of O&M [4] may use the concepts provided in this standard to identify domain-specific relationships between a sampling feature and the ultimate sampled feature.

The model concepts may be used to describe the relationships of observation results to the hydrologic feature of interest they are meant to represent and in this way to link spatial and non-spatial hydrologic data. In particular, this model will support interoperability at semantic level through providing a standard terminology, and formalizing the information relationships and constraints on behalf the data sharing and system communication.

this allows links to be implemented in a standard way between resources on the Web. For example, a discovery service such as a catalog of catchments could use the concepts defined here to provide machine interpretable pointers to services that provide differing geospatial representations of the same catchments as well as data from water quality and water quantity observations that characterize some aspects of the catchments. A client could then use those links for automated workflows and data product generation.

Figure 1: HY_Features in the context of the OGC Abstract Specifications

# Conformance

This standard defines the conceptual model for identification of hydrologic features and their fundamental relationships. The conceptual model may be used in two ways: a) to derive implementation classes for data exchange or b) via reference (referred to here as mapping) from terms used in an implementation to the equivalent terms in the HY_Features model. The form of such a mapping is not specified in this standard, but in general there is correspondence expected between particular implementations of hydrologic features and realized HY_Features concepts. There will be an exact correspondence or a correspondence to a specialization of a HY_Features class with narrower scope. Since no technical semantic mapping standard is supported by the OGC standards baseline at this time, only the expressivity requirements of mappings are specified in relevant conformance clauses.

Requirements for OGC HY_Features standardization target types are considered:

- Encodings (such as OWL and RDF) of the HY_Features conceptual UML model described in this standard and

- Application schemas formally mapped to HY_Features concepts including Feature Type classes and associative relationships.

For brevity, the terminology implement is used to indicate either a direct encoding or existence of a formalized mapping that would enable a client, at run-time, to determine that a particular implementation class implements a specific HY_Features concept.

Part 2 of this standard (a future document) is intended to provide an OWL encoding of the HY_Features application schema, whereby the mapping from the implementation to this specification will be formalized through use of encoding rules that allow direct correspondence of schema elements with the UML elements defined.

Conformance with this standard shall be checked using all the relevant tests specified in Annex A (normative) of this document. The framework, concepts, and methodology for testing, and the criteria to be achieved to claim conformance are specified in the OGC Compliance Testing Policies and Procedures and the OGC Compliance Testing web site.

In order to conform to this OGC™ interface standard, a software implementation shall choose to implement any one of the conformance levels specified in Annex A (normative).

All requirements-classes and conformance-classes described in this document are owned by the standard(s) identified.

# Normative References

The following normative documents contain provisions that, through reference in this text, constitute provisions of this document. For dated references, subsequent amendments to, or revisions of, any of these publications do not apply. For undated references, the latest edition of the normative document referred to applies.

ISO 19101:2002, Geographic Information—Reference Model

ISO/TS 19103:2005, Geographic Information — Conceptual schema language

ISO 19107:2003, Geographic Information — Spatial schema

ISO 19108:2006, Geographic Information — Temporal schema

ISO 19109:2006, Geographic Information — Rules for application schemas

ISO 19111:2007, Geographic Information — Referencing by coordinates

ISO 19115:2012, Geographic Information — Metadata – Fundamentals

ISO 19133:2005, Geographic Information — Location-based services – Tracking and navigation

ISO 19148:2012, Geographic Information — Linear referencing

ISO 19156:2011, Geographic Information — Observations and Measurements

# Terms and Definitions

This document uses the terms defined in Sub-clause 5.3 of [OGC 06-121r8], which is based on the ISO/IEC Directives, Part 2, Rules for the structure and drafting of International Standards. In particular, the word SHALL (not "must") is the verb form used to indicate a requirement to be strictly followed to conform to this standard.

For the purposes of this document, the following additional terms and definitions apply.

- application schema

Conceptual schema for data required by one or more applications [ISO 19101].

- boundary (line)

Geometric representation of the (topological) catchment boundary, usually a geometric composite curve

NOTE: This definition references the  definition of a divide  described as summit or boundary line [9].

- catchment

A physiographic unit where hydrologic processes take place. This class denotes a physiographic unit, which is defined by a hydrologically determined outlet to which all waters flow. While a catchment exists, it may or may not be clearly identified for repeated study.

NOTE: This approach considers the catchment concept to be the basic unit of study in hydrology, water resources management, and environmental reporting. The approach is meant to be holistic, referring to the continuous interaction of surface and subsurface waters within a catchment, even if a particular representation of the catchment refers to only surface or subsurface aspects of the catchment. Special subtypes such as drainage basin and groundwater basin may be defined with a particular application.

The synonym use of the terms catchment and catchment area as documented in the WMO/UNESCO International Glossary of Hydrology (which is the key reference for the definitions in the HY_Features model does not clearly distinguish between the catchment concept and its geometric representations such as catchment area, nor between catchment and its possible specializations like drainage basin and groundwater basin.

The essential logical separation of concerns between a common concept and its representation (interpretable data) is dealt with by defining separate features for the catchment and its representation and may be understood as a refinement of the complex, ambiguous definition given in the glossary. See also clause 5.4 of this standard on the use of WMO terminology.

- catchment area

Two-dimensional (areal) topological feature realizing the logical catchment (idea) in terms of a face bounded by catchment boundary and flow path edges. The concept of a face bounded by edges is described in detail in the ISO topology model [ISO19107].

NOTE: Hydrologically, the catchment area references the catchment having a common for its runoff [9], through using the synonym catchment area to express the two-dimensional representation of the catchment [4.2].  The catchment area is usually represented as a geometric surface, and the measure of the catchment area may be denoted as surface area (if required).

- catchment boundary

One-dimensional (linear) feature realizing the logical catchment (idea) in terms of a topological edge bounded by inflow and outflow nodes, and associated with catchment faces inside of the boundary. The concept of an edge bounded by nodes is described in detail in the ISO topology model [ISO19107]. A catchment boundary is usually represented as boundary line.

NOTE: Hydrologically, the catchment boundary references the divide separating adjacent catchments [9].

- catchment topology

Topological closure of catchment and outfall hydrologically determined through the union of catchment and its common outlet.

- channel

Natural or artificial waterway, clearly distinguished, which periodically or continuously contains moving water, or which forms a connecting link between two bodies of water [9].

- channel network

Aggregate of depressions and channels which continuously or periodically contain water.

- contour (line)

Geometric representation of the (topological) water edge, usually a set of curves (multi-curve).

NOTE: This definition references the  definition of a contour-line on a map indicating the locus of points at which a certain property is constant (e.g. elevation, salinity) [9].

- cross section (of a stream)

Section of a stream at right angles to the main (average) direction of flow [9].

- cross Section (of a stream bed)

transversal section of a stream bed in a vertical plane.

NOTE: This definition references the  definition of a bed profile describing the shape of a stream bed  in a vertical plane [9].

- data

Documented value of some characteristics of a real-world phenomenon.

- data set

Data compiled and arranged into a set.

- data product

Data set compiled for a specific purpose, e.g. for global dissemination using Web services.

- dendritic catchment

Catchment wherein all waters flow to a single common outlet. A dendritic catchment is permanently connected to others in a dendritic (tree) network.

NOTE: This definition references the most common drainage pattern of streams ultimately flowing into the ocean after joining together at confluences into larger streams.

- depression:

Landform lower than the surrounding land, containing water, or not.

- domain feature

Feature of a type defined within a particular application domain. [ISO 19156].

- endorheic (drainage)

Draining (ultimately) into interior catchments [9].

- exorheic (drainage)

Draining (ultimately) into the ocean [WMO, 2016).

- feature

Abstraction of real-world phenomena. [ISO 19101]

- flow path (also flowpath)

One-dimensional (linear) feature realizing the logical catchment (idea) in terms of a topological edge bounded by inflow and outflow nodes, and associated with left-bank and right-bank catchment faces. The concept of an edge bounded by nodes is described in detail in the ISO topology model [ISO19107].

NOTE: Hydrologically, the flow path references the path line described by a moving particle of water [9].

- flow line (also flowline)

Geometric representation of the (topological) flow path, usually a geometric curve.

NOTE: This definition references the  definition of a path-line [9] through using the synonym flow line to express the geometric representation of the flow path.

- hydrographic network

Aggregate of rivers and other permanent or temporary water bodies, including lakes and reservoirs [9], standing in depressions or moving in channels.

NOTE: Not to be confused with the network of hydrological stations and observing posts.

- hydrography

Science dealing with the description and measurement of open bodies of water. [9]

NOTE: In this context, hydrography refers to the description of surface-water bodies. Its measurement in terms of surveying, e.g. for navigational purposes, is not in the concern of the HY_Features model.

- hydrologic complex

Collection of separate hydrologic features forming a closed, hydrologically determined system where the union of catchment(s) and a common outlet (conceptualized as an outfall) is realized by a ‘complex’ of hydrologic features. This concept is rooted in the idea of topological closure of catchment and outfall such that a realization of the logical catchment is always of higher topological dimension than the realization of the corresponding outfall. For example, a linear flowpath realizing a catchment may be understood as an edge between inflow and outflow nodes; the areal realization of a catchment as a face bounded by linear inflow and outflow.

NOTE: Hydrologic determination means that any catchment has a common outlet for its runoff. Any place to which water may flow can be associated with a corresponding catchment, even if catchment and outfall may be unknown or not represented in a particular application.

- hydrologic feature

Feature of a type defined in the hydrology domain, whose identity needs to be maintained and tracked through a processing chain from measurement to distribution of hydrologic information.

- hydrologic realization

Hydrologic feature that realizes a logical concept (idea) within the hydrologic complex. Example: Hydrographic network realizing the logical catchment as network of one-dimensional flowpaths.

- hydrology

Science that deals with the waters above and below the land surfaces of the Earth, their occurrence, circulation and distribution, both in time and space, their biological, chemical and physical properties, their reaction with their environment, including their relation to living beings. [9]

- hydrometric feature

Feature of a type which denotes a physical structure intended to observe properties of a hydrologic feature.

NOTE: This definition references the  definition of a hydrometric station at which data on water in rivers, lakes or reservoirs are obtained on physical and chemical properties of water [9]. Used to sample a hydrologic feature, a hydrometric feature may be considered a sampling feature of defined in the ISO Observation model. A sampling feature is described in general in ISO 19156: Observation and Measurement, the special monitoring point of hydrologic observation is described in the OGC WaterML 2.0: Part 1- Timeseries standard [4].

- hydrometric network

Aggregate of logically connected monitoring stations used for hydrologic observation.

NOTE: This definition references the  definition of a hydrological network hydrological stations and observing posts situated within catchment in such a way as to provide the means of studying the hydrological regime [9].

- hydrometry

Science of the measurement and analysis of water including methods, techniques and instrumentation used in hydrology. [9]

- interior catchment

Catchment wherein all waters are collected endorheic; an interior catchment is not connected to other catchments.

NOTE: This definition is rooted the blind drainage pattern of collecting water in sinks or lakes not connected to streams [9].

- linear element

Linear geometric representation, which forms the axis (shape) in a linear coordinate system.

Anything which can be measured using a linear referencing [ISO 19148].

- linear coordinate system

One-dimensional coordinate system in which a linear feature forms the axis [ISO 19111].

- longitudinal section (of a stream)

Vertical section of a stream in longitudinal direction.

NOTE: Note: This definition is rooted in the definition of a longitudinal section along a channel at its center line [9], but generalized for all types of vertical section along a line.

- longitudinal section (of a stream bed)

Longitudinal section of a stream bed in a vertical plane.

NOTE: This definition references the  definition of a bed profile describing the shape of a stream bed  in a vertical plane [9].

- main stem

Main course along which a stream flows, usually the main section of a stream from headwaters to the mouth, except tributaries.

- mapping

Establishing a semantic relationship between particular implementations of a common concept and the realized normative concept using a formalism that specifies how elements from a source model may be transformed to a target model.

- named feature

Feature identified by a name.

NOTE: Hydrologic features and their real-world representations have names within common experience, but may have different names in their cultural, political and historical contexts.

- outfall

Logical outlet of a catchment. The outfall represents the logical place where a catchment interacts with another catchment. The outfall denotes the union of catchment and its common outlet which means that the logical catchment in general exists with an outfall, and the logical outfall cannot exists without a corresponding catchment.

NOTE: The existence of a logical outfall which is unknown, un-realized, or not represented, in a given implementation is termed ‘nillable’ in this standard. For example, a logical outfall exists in the form of flow to the subsurface or atmosphere but is unknown in a surface-water hydrology implementation.

- representation

Any processible data, data set, or data product, which can be used in the place of an existing feature concept.

- river positioning system

Linear system used to reference indirect positions along a watercourse.

- storage (of water)

Impounding of water in surface or underground reservoirs, for future use. [WMO,, 2016]

NOTE: Storage refers to a water body in terms of a usable water resource. The management of the reservoir as human action with the objective to efficient and sustainable use the resource, is not in the scope of the conceptual model. Yet, often an indication is required whether a water body is used for storage.

- stream

Water, generally flowing in a natural surface channel, or in an open or closed conduit, a jet of water issuing from an orifice, or a body of flowing groundwater [9].

- water body

Mass of water distinct from other masses of water [9].

- water edge

One-dimensional (linear) feature realizing the logical catchment in terms of a topological edge bounded by inflow and outflow nodes. The concept of an edge bounded by nodes is described in detail in the ISO topology model [ISO19107].

NOTE: Hydrologically, the water-edge references the shore-line of intersection between a water body stratum and the confining depression, or channel [WMO. 2016].

- watercourse

Natural or man-made channel through or along which water may flow [9], including large interstices in the ground, such as cave, cavern or a group of these.

# Conventions

This section provides details and examples for any conventions used in the document. Examples of conventions are symbols, abbreviations, use of XML schema, or special notes regarding how to read the document.

## Identifiers

The normative provisions in this specification are denoted by the URI:

http://www.opengis.net/spec/{standard}/{m.n}

All requirements and conformance tests that appear in this document are denoted by partial URIs which are relative to this base.

## Symbols (and abbreviated terms)

CHy 		WMO Commission for Hydrology

GML 		Geography Markup Language

GRDC 	Global Runoff Data Centre (GRDC)

GWML2 	GroundwaterML 2

HDWG 	OGC Hydrology Domain Working Group

ISO 		International Organization for Standardization

OGC 		Open Geospatial Consortium

OWL 		Web Ontology Language

RDF		Resource Description Framework

UML 		Unified Modeling Language

WaterML 2	working title for an indented suite of water-related OGC standards

WIS 		WMO Information System

WIGOS 	WMO Integrated Global Observing System

WMO 		World Meteorological Organization

XML 		eXtensible Markup Language

## UML notation

Most diagrams that appear in this specification are presented using the Unified Modeling Language (UML) static structure diagram, as described in Sub-clause 5.2 of the OGC Web Services Common Implementation Specification (OGC 04-016r2). UML classes are named in UpperCamelCase and property names in lowerCamelCase.

## WMO Terminology

The HY_Features model uses—as far as possible—terminology recommended by the WMO Commission for Hydrology for use in the WMO Member countries. The key reference is the "International Glossary of Hydrology" [9] , a joint publication of the WMO and the UNESCO (hereinafter referred to as WMO/UNESCO glossary of hydrology). Wherever appropriate, terms from this glossary are applied to the feature concepts in this standard to capture meaning and contextual relationships. The synonym approach widely used in this glossary is interpreted in that way that glossary terms, when explicitly defined as synonyms, were defined as such even if they are not synonymous in every respect. Differences in terminology were explored through reconciling the explicit definitions documented in the glossary with aspects reflected in various data sets and products in use. The accepted terms were augmented with the relationships inferred from other terminology in order to define complex terms that do not clearly distinguish between a specific logical concept and its geometric representation or between a general term and its specific conceptual meaning. The definitions used in the conceptual model described in this standard may be understood as a conceptual refining of the definitions given in the WMO/UNESCO Glossary of Hydrology.

Though basically rooted in the definitions given in the WMO/UNESCO Glossary of Hydrology, for the purpose of testing the applicability of the conceptual model in the context of surface water hydrology, the definitions applied to hydrologic features defined in this standard refer to surface water.

Some requirements classes defined in this standard refer to the Scoped Name concept of ISO 19103: Conceptual Schema. Intended to form a basis for information and data sharing in the community of the WMO member countries, the Scoped Name should reflect a name endorsed by the WMO.

## Naming convention

The HY-prefix used in the UML model follows the ISO naming conventions for UML elements. There is no explicit requirement to use this name in an implementation for the same semantic elements, but it is required to provide an explicit mapping between terminology used and HY_Features to unambiguously indicate hydrologic feature concepts to support unambiguous interpretation in cross-domain applications. While there is not a recognized standard method for recording mappings between abstract element names and implementations; it is expected that semantic and system interoperability will be facilitated by making such mappings available as part of a dataset's documentation.

# Clauses not Containing Normative Material

## The abstract idea of the hydrology phenomenon

Processes that continuously deplete and replenish water resources cause or result in a wide range of phenomena that are the subject of monitoring, modeling and reporting in hydrology and related sciences. These distinctly named or otherwise uniquely identified real-world hydrologic phenomena are conceptualized and referred to as hydrologic features in this standard. This standard applies to data and data products that represent these hydrologically defined features.

Figure 2: Processes of the Hydrologic Cycle

Water is moving from the atmosphere to the Earth and back to the atmosphere due to the processes forming the Water Cycle (Figure 2). Water from precipitation reaching the land surface is accumulated in water bodies occupying empty space on the land surface or in water bearing formations of soil and rock. Excess water overflows these formations and is driven downhill by gravity. Water flowing over soil or rock causes erosion to occur. This erosion tends to concentrate flowing water into water bodies that flow downhill using a connecting system of channels intersecting other water bodies along their way to a common outlet, conceptualized as a potentially complex outfall feature.

Looking back upstream from the outfall, the corresponding catchment feature can be described as a main linear flowpath feature, an areal feature, a boundary feature that encompasses the drained area, a network of water body features, a network of channel features, or a network of hydrometric station features, all representing the catchment. The concepts and terminology used in the preceding sentences form the conceptual and semantic basis for this standard.

As described above, the most general abstraction of the hydrology phenomenon is the catchment. A catchment is a recognized unit of study where hydrologic processes form physiographic features that are realized in various data products. In this specification, the term realization is used to imply that a catchment is recognized and identified for the purpose of referencing it by name or within a topology of other catchments. The term representation is used where geometry is used to represent a catchment's physical features. Using these definitions, we can say that a catchment is an unrealized conceptual feature that can be realized by features for use in topological networks and features with explicit geometry.

Depending on application and (spatial) scale, the same catchment may be realized in many different ways. The following describe different examples where this multiple realization concept is important:

- The most typical example is that scaling a map-visualization up or down leads to multiple more or less detailed representations of the same realized hydrologic feature.

- Analyses and reports from various disciplines reference differing spatial representations of the same conceptual catchment.

- Some applications require cartographic (visual) representations while others are focused on topological (network connectivity) realizations only (Figure 3).

- A catchment may be represented geometrically by streamlines, drainage area or the bounding polygon, or realized topologically as a face, edge, and/or nodes.

Catchments can be realized as geographic streams and outfalls, simplified geographic stream representations, schematics of streams and outfalls, an aggregate of catchments, simplified representations of catchment aggregates, or schematics showing how catchments relate.

The concept of catchment used in this standard is that it is described physically as illustrated above. It can also be described as the recognized unit of water resources assessment and management across administrative jurisdictions. In multi-stakeholder collaboration and cross-domain research projects, the catchment is recognized as a shared monitoring and reporting unit, in that monitoring stations are usually on a river in a recognized catchment, attributes of the catchment may be in reference to its geometry, and its inflow and outflow are network locations connecting it to other catchments in a topology. Examples of catchments include "Hydrologic Unit Code" (HUC) catchments defined by the U.S. Geological Survey for research and regulatory data systems to use as a reference [7] and "River Basin Districts" of the European Water Framework Directive [1].

Figure 3: Illustration of multiple representations of a catchment

## Catchment and realizations of the catchment concept

The core concept of the HY_Features model is that a study of the Hydrology phenomenon will reference common conceptual entities of the real world such as catchment, water body, channel, or stream gaging site, through the use of features according to a specific model (as per ISO 19109 General Feature Model). Depending on the field of study or application, complex hydrology phenomenon features may be realized in many ways. This leads to the idea that each study of a catchment may portray it in a particular way leading to multiple realizations of the same conceptual entity. Depending on the scientific concern, the specific hydrologic feature is understood as one of many potential realizations of the conceptual entity being studied.

Figure 4: Idealized catchment. This idealized catchment diagram is used to represent the catchment concept with no emphasis on any realization or representation. In the diagrams below, aspects of this diagram are highlighted to emphasize particular concepts of the HY_Features conceptual model.

Across hydrologic disciplines the concept of catchment defined in this standard is the recognized conceptual unit of study where hydrologic processes take place. Catchment boundary, catchment area, and linear flowpath are the most common realizations of a catchment, and are widely used to create cartographic views of a catchment. Map layers that separately visualize the hydrographic network of water bodies (typically as blue lines for small rivers and blue polygons for larger rivers and lakes) and of channels (typically lines displaying a drainage pattern indicating the path flow may follow), or a network of hydrometric monitoring stations, are usually combined to portray the represented catchment as a whole. Such cartographic data products are usually exposed in map services, while the water body or channel features are provided with an appropriate geometry and location usually in features services. To illustrate these ideas, Figure 4 shows an idealized catchment with different graphical representations highlighted.

Figure 5: Multiple graphical representations of a catchment (from top left to bottom right): a) Catchment boundary, b)   Catchment area, c) flowpath of catchment d) network of sub catchments, e) cartographic view, f) abstract flow paths, g) hydrographic network, h) network of logically connected monitoring stations.

Each of these graphical representations is a different way of looking at the catchment and its interaction with other features. It is generally not possible to inspect a particular realization and understand all characteristics of the catchment because different types of features are often realized using identical data models. For example, a map showing a set of catchment polygons may display an aggregate of sub-catchments, or a collection of catchment polygons that overlap each other occupying the same space. In order to understand what catchment concept is being realized, the more detailed information about their type and role in a data model needs to be declared. These relationships and potential constraints may be declared between a high-order catchment and the catchments generally nested therein, or between an aggregate network of catchments that encompasses nested catchments without any overlap, or the upstream-downstream direction of flow for a set of catchment polygons.

In some cases, a catchment may be realized as an identified feature without specified geometry that takes part in a topological network of catchments. This is common, for example, in data systems supporting hydrologic modeling. Catchment attributes needed to parameterize a model, may be generated using various geospatial data sets, but the geometric information is not necessary for the actual modelling. In applications like this, and in data systems supporting such applications, a catchment can be realized by an identifier, attributes, and topological connections that are not necessarily the same as would be implied by geometric representation of a catchment. That is, while a catchment has a geometric representation that is an area, which would imply it is a topological face, the catchment can have a non-geometric realization that is a topological edge which conveys flow from inflow to outflow. This flexibility, where a realization of a catchment feature does not need to have an explicit geometric representation allows a decoupling of visual and conceptual concerns with the ability to explicitly link data products that would not otherwise be easily related.

## Catchment hierarchy and network topology

Catchments are usually organized in hierarchical networks to reflect their continuous interaction.   This provides the ability to aggregate catchments, and to trace catchment networks up- or down-stream.

Referencing the hydrologic determination through a common outlet, the catchment forms the logical link that connects  a catchment’s inflow and outflow whereby these may be joined from diffuse inflow, or spread outflow.  In geometric representations, topological connectivity is typically indicated by adjacent polygon edges, nesting of polygons, or through connection of linear features at nodes. It is common in hydrology to describe connected catchments using geo-schematic ‘flow lines’ which realizes the logical union of catchment and common outlet in terms of a topological edge bounded by inflow and outflow nodes.  However, since geometric representation of hydrography serves many purposes and may not be needed at all, it is not appropriate to rely only on geometry as the basis for topology. Instead, topology can be expressed as relationships between conceptual features of a particular type which may or may not have explicit geometric representations.

Figure 6 (C1-C5, from left to right): C1, Typical catchments with one inflow and one outflow each; C2, Joined (conjoint) catchments flowing into a single downstream catchment; C3, catchments joining in a water body or wetland with no clear network; C4, catchments joining through intermittent or subsurface flows C5, catchments that join through areas of complex or braided channels.

In a network of catchments, morphological detail may be specified in many ways. Inflows are generally conceptual in headwaters, and outflows are often complex where water flows out of a network. As shown in Figure 6 catchments may connect through simple confluences (C1), water bodies or wetlands (C3), intermittent or subsurface flows (C4) or complex braided streams (C5). Like diffuse (multiple) inflow may be logically joined in a ‘conjoint’ catchment (C2),  spread (multiple) outflow may be joined in a catchment flowing out at a single, logical outflow. Although these cases require different geographic representations, they can be represented using the same topological  pattern of the catchment bound with its common outlet. Since all these cases can be specified using simple catchment topology, no special treatment is required to handle the variation of flow processes.

### Hierarchy of catchments

Any catchment may be nested or aggregated in a larger containing catchment or split into multiple sub units forming a hierarchy of catchments. Two types of catchment hierarchy are supported in HY_Features: basic nesting and dendritic aggregation: 1) Basic nesting allows any catchment to have a reference to a containing catchment (Figure 7). This allows collections of sub-catchments to be grouped into larger units, but does not define any particular interconnections between these sub-catchments. 2) Dendritic hierarchies are collections of catchments with simple topological relationships that allow determination of contribution of flow to downstream catchments. To reflect the organization of catchments in dendritic networks, a special dendritic catchment is defined that permanently contributes exorheic flow to a receiving catchment (Figure 8), and an interior catchment of endorheic flow that contributes temporarily to a receiving catchment.

Figure 7: Catchment hierarchy – A catchment (dark grey), may be nested within a containing catchment which is another catchment (light grey)

Figure 8: Catchment hierarchy – A catchment (C1, C2, or C3) may be part of a simple dendritic network of catchments which is also a catchment.

### Topological network of catchments

Given the idea that a non-geospatial schematic representation of a feature can show its connection in a topological network, a catchment can be thought of as a hydrologic unit whose terrain and morphology results in a topological link between inflow and outflow. The red line in Figure 9 illustrates how a single catchment, represented by a boundary, an area, a stream network, and a main flowline, could also be represented using a single line that is not geospatially representative but is a valid schematic representation of the connection between inflow and outflow.

Figure 9: A catchment area (grey) and a flowpath connecting inflow to outflow (red) depicting a definable unit where hydrological processes take place.

Figure 10: Catchments contributing to an identified outflow node. Note that some catchments contribute to a common outflow node.

Figure 11: Catchments receive inflow via an identified inflow node. Note that nodes are not necessarily geographic features, but are rather nodes in a graph representation of the river network.

The catchment C1, shown in Figure 10, contributes flow to outflow node n1. Node n2, shown in Figure 11, contributes inflow to catchment C1. Networks of catchments can be constructed using this construct and the idea that two or more catchments that flow into the same downstream catchment, first flow to the same outflow node which is the inflow node of the receiving downstream catchment. In a network of dendritic catchments, outflow nodes on the network serve only one receiving catchment.

Nodes may seem to stand alone as points of interaction in the network, but in reality, they are (potentially complex) catchment outlets, denoted as outfall features. Whether an outfall (node) is referred to as an inflow node or outflow node is always in reference to a particular catchment. This means that an outfall serves as the outflow node of some contributing catchment(s) and the inflow node of some receiving catchment(s). In this standard, the inflow and outflow role names of the conceptual outfall (node) are used to unambiguously describe the role of an outfall with respect to a catchment.

In a dendritic network, the outflow that one or more dendritic catchments flow to (which is not necessarily a single geometric point) must contribute to one and only one receiving catchment, unless it is a terminal catchment. Given that the dendritic catchment is defined as a special type of the more general catchment concept, it inherits the general nesting as defined for the catchment as shown in Figure 7.

Being topological nodes, inflow and outflow have no explicit positions but are potentially complex catchment outlets. In fact,  inflow and outflow may be represented by a complex geometry with multiple parts. This is important in the case of a catchment that contains a broad river bottom with complex braided channels and two or more primary inflows. There may be no clear way to identify an inflow location, but from a topological perspective each contributing catchment can be said to contribute to the same outflow and that diffused inflow can be said to contribute to the catchment in question (Figure 12). There might be two ways that catchments with multiple inflows can share a common outlet: 1) as separate catchments, each realized as a flowpath connecting a single inflow and the common outflow, or 2) as a non-divided catchment contributing as a whole to the outflow node. The latter case, referred to here as a conjoint catchment, requires to determine the main flowpath, or to perform linear referencing (along the main flowpath). However, a conjoint catchment is often easier to delineate and more convenient. Note that in this case, the complexity required to support geospatially accurate linear referencing may be lost in the interest of easy network navigation capabilities.

Figure 12: Encapsulation of non-dendritic stream network topology. The left figure shows a case where it is not possible to determine to what extent flow from catchment F contributes to catchments E, B or C. The right figure shows how catchments E, B, and C can be aggregated so nodes N2 and N3 are treated as a single virtual inflow node, so that all the flow from catchments D and F accumulate in the resulting catchment X.

It is worth noting that non-dendritic networks are often realized as a dendritic catchment network by introducing joint catchments that contain the non-dendritic parts. Figure 12 shows an example of a non-dendritic stream network, where it is not possible to determine to what extent flow from catchment F contributes to catchments E, B or C (Figure 7, C1). Aggregating  the catchments E, B, and C  (Figure 7, C2) and collapsing the nodes N2 and N3 into a single virtual inflow (node), will accumulate all the flow from catchments D and F in the resulting catchment X contributing inflow into catchment A via the node N1 (Figure 7, C3). Using this encapsulation approach, catchments can be represented using a simple tree structure where an upstream-downstream relation can be built without the need for complex hydrography between inflow and outflow nodes.

## River reference system

Considering any identified location on a network as the outflow node of a contributing catchment, or the inflow node of a receiving catchment, an arbitrary new location can be placed on the network in reference to the existing outfall up- or downstream. Understanding such a reference as a positioning "along a river", the linear flowpath representation of a catchment can be used for linear referencing. HY_Features defines the concept of a river reference system which has three components: 1) an origin at an inflow or outflow outfall; 2) a shape defined by the flowpath of a catchment that starts at the origin; and 3) a linear distance or relative (percentage) measurement system. Each catchment has its own reference system which can be combined as necessary using catchment topology; each river reference system must have one outfall (origin) and one representing flowpath (shape).

Provided that the location to be placed is declared to be outfall of a catchment, an already existing catchment will be split into catchments upstream and downstream of the ‘new’ location. The upstream catchment is realized by a flowpath (shape) starting at the location to be placed (origin), and directed to the already located inflow of the now split catchment;  the downstream catchment is realized by a flowpath shape starting at the same location, but directed to the already located outflow of the now split catchment.  The unknown position of the ‘new’ location is provided relative as the distance from there to the reference location, i.e. as length of the flowpath in direction to the known inflow or outflow. In this way a position may be assigned to any outfall. Figure 13, and Figure 14 illustrate how a newly introduced network location can be located downstream of a catchment inflow, or upstream of a catchment outflow.

Figure 13: Position (yellow dot) downstream of a reference point (red dot). While referenced positions (P) are usually referenced to permanent locations like confluences (r2), they can also be considered to be the outflow of contributing catchment and thus, the origin of a river reference system of their own.

Figure 14: Position (yellow dot) upstream of a reference point (red dot). Typically, r1 would be the origin and P, the referenced point; however, the river reference system model allows for the referenced point to be the origin of its own river reference system.

Alternatively, the position of a new location can be determined interpolative as a percentage of the known distance between two already located outfalls bounding a flowpath in upstream and downstream direction, whereby the origin where the flowpath starts is set at one of these, and directed to the other outfall upstream or downstream of this. This approach may be used in the case that the new location itself is not declared to be an inflow or outflow of a catchment, as shown in Figure 15.

Figure 15: NEW  Position (yellow dot) between two a reference points (red dots).

## Hydrographic and hydrometric networks

Hydrographic data is commonly organized into networks. HY_Features includes catchment realizations for the hydrographic network of waterbodies, channel network connected conveyances, and hydrometric network of monitoring stations. The following sections first describe the individual network types uniquely, then describe how these networks relate to each other in the context of the overall HY_Features model. The main function of these network models is to allow an aggregate of hydrologic features at one scale to act as a single encapsulated entity at another scale.

### Hydrographic and channel networks

Following the definitions in the UNESCO-WMO "International Glossary of Hydrology" [9] a water body is understood as the mass of liquid water accumulated on or below the land surface as a body of flowing water, which in some parts may have stagnant water that is not moving or flowing. The water body concept formalized in this specification is consistent with this definition, but focuses on surface-water bodies only. A conceptual model capturing the specifics of bodies of groundwater as well as aquifers containing groundwater are provided by the WaterML2 Part 4 - GroundwaterML2 specification [6].

A watercourse is commonly understood as a natural or man-made channel through or along which water may or may not flow [9]. A water body is generally a flowing body of water contained by a natural watercourse, but could also be in a manmade open or closed conduit [9]. Given that the channel network (or drainage pattern) exists independent of whether it contains water, HY_Features conceptually separates natural and manmade hydrographic features into the body of water and the unit that contains the water body. This distinction is helpful to separate concerns of hydrology--studying the occurrence, accumulation, and circulation of water--and hydraulics--focused on the analysis and design of watercourses. Note that flow-through lakes and lakes without outflows are considered to be water bodies whose container is modeled as a surface depression.

Water bodies with their associated water courses can be aggregated into hydrographic and channel networks identified as the hydrographic or channel network of the catchment the network drains. Individual waterbodies can be described with vertical cross- or longitudinal sections. A water body may also be stratified into distinct horizontal layers for distinct thermal, salinity, oxygen, nutrient, or storage characteristics. Water may be stored and managed in reservoirs for future use, regulation, or control. Conceptually, each water body accumulating water may be managed as a reservoir. To model reservoir storage at multiple reservoir stages or elevations, HY_Features includes an association between a reservoir and its water-body stratum. The concept of stratum could also be applied to bathymetric contours of a lake to describe the container of a lake water body.

To place waterbodies topologically or geographically in a network, the HY_Features model provides a mechanism to associate waterbodies with a realization of a catchment outfall. In the case of a hydrographic network that realizes an entire catchment, the HY_Features catchment relationships can be used for both the larger catchment realized by the whole hydrographic network and the catchment(s) whose outfalls are used to locate waterbodies within the hydrographic network. This builds the idea outlined in section 6.3 in that every identified water body has a conceptual catchment that can be arranged topologically within a catchment network and is described in greater detail in section 6.5.3.

### Hydrometric networks

Water bodies are observed using monitoring stations which are typically physical locations with a well-established cross section they monitor. It is common practice to locate a monitoring station in relation to local landmarks and permanent reference points along a stream or in relation to the outlet of the monitored water body. The river reference system, described in section 6.4, gives a mechanism to locate such points in relation to establish catchment outfall locations such as confluences. When aggregated into a network, monitoring stations within a catchment can be said to make up a logically connected hydrometric network which monitors a catchment.

The purpose of HY_Features hydrometric stations and network is to support linking identified monitoring stations, (thought to be the sampling feature in terms of the observations and measurements standard) and a river or other hydrologic feature (thought to be the domain or sampled feature in terms of observations and measurements). The monitoring feature role of hydrometric features is unique in contrast to all other outfall realizations in HY_Features, which would be used as observed domain features in observations and measurements.

### Hydro Network: hydrographic network, channel network and station network - different views of catchment topology

Maps displaying a representation of a catchment are very common in hydrology research and engineering. Depending on the scientific concern, different aspects of the hydrology phenomenon are represented using application specific map symbols. Respecting the separation of scientific concerns, HY_Features distinguishes between these by defining separate realizations of the catchment and outfall concepts which may be simple points, lines, or polygons, and may be complex aggregate networks made up of collections of point, line, or polygon representations. The need to support such collections across scales is supported through the general concept of the Hydro Network described below.

The holistic idea of catchment, where a conceptual catchment has many real-world realizations, allows us to consider realizations of a catchment that correspond to different phenomena as variations of a common catchment pattern. Using this approach, the relationship of a catchment to a set of catchments can be transferred to any representation of that catchment regardless of the geometric representation in a particular data product. For example, an entire hydrographic network representation of a catchment can have simple relationships to the hydrographic network of its up and downstream catchments if the network itself is understood to adopt basic catchment characteristics.

This standard defines a hydrologically determined topology model of directed outfalls, acting as inflow or outflow nodes, and the catchment which can act as the link between them. This topological catchment network pattern can be transferred to context-specific network realizations such as hydrographic networks of water bodies or channel networks that may convey water, as well as to their various representations as (poly)lines and polygons. For example, a fixed landmark on a water body, a cross-section separating a watercourse, or a station along the network, can be considered to be outfalls (outflow nodes) of a contributing catchment (link); a flowpath representing a catchment may be drawn from an ‘inflow‘ node to the ‘outflow‘ node linking the nodes through the represented catchment. As with the hydrographic (water body) and channel (conveyance) network, the abstract catchment topology model can be applied to any logical or virtual network, e.g. a network of logically connected hydrometric stations.

Figure 16: Cyclic nature of the HY_Features basic model

Figure 16 illustrates the circular relationship between five functional components of the HY_Features model: the catchment concept, catchment realization, hydro networks, waterbodies, and outfalls. The cyclic nature of the basic model supports crossing scale through nesting of more or less detailed catchment realizations.

### Flow Path

In practice, catchments are associated with a single ‘main-stem’ flowpath (Figure 5c). Some elevation-derived hydrographic datasets define one flowpath per catchment such that every line connecting two confluences has a single associated drainage area. Others have an identifiable ‘main-stem’ river that flows from headwaters or inflow to outflow. In either case, the main flowpath typically carries the name of the catchment and its outlet is considered the mouth of the river or catchment. HY_Features includes a simple linear realization of the catchment concept that is meant to encompass the ‘main-stem’ idea.

The flowpath concept is associated with a hydrographic or channel network in that any hydrographic or channel network can have one identified flowpath connecting its inflow to outflow node. In this way, at large scales, complex networks of channels can have one main-stem flowpath made up of many smaller scale reaches. At very small scales a network made up of one reach also has a single main flowpath. In both cases, the flowpath is associated with a catchment and should be understood to be the one linear connection between inflow and outflow node. Because hydrographic and channel networks are aggregates of waterbodies and channels, it follows that the flowpath concept is related to waterbodies and channels through the networks that aggregate them.

## Discussion of purpose and relation to ISO standards baselines.

The intention of this standard is to provide the conceptual basis for linkage of identified hydrologic features encoded in data products. Given that any given real-world hydrologic entity can be a subject of study over time, between organizations, and across disciplines, the identified feature is often represented in many ways in differing data products. The ability to reference such features between data products requires a scale-independent, nestable catchment model that can be visualized using common geometric shapes to represent the catchment in geographic maps and geo-schematic views. In correspondence with the fundamentals of hydrology, catchment topology is determined through the common outlet to which all waters flow within a bounding watershed. This is in contrast to use of geometric representation for determination of catchment topology (although geometric representations may of course assist in the determination of a set of common outlets).

The hydrologic feature model described in this standard describes the hydrosphere as a scale-independent hierarchical network of catchments. The defined catchment topology follows hydrologic patterns governed by physical laws in a way that is independent of any possible geometric representation. The pattern can be replicated multiple times to build hierarchical networks of hydrologically discrete catchments. In practice, the highest-level of this hierarchy reflects a river system flowing into a sea or an internal sink and the lowest level is the smallest defined unit of study. Depending on the scientific concern of a study, the hierarchical network of catchments will be used to describe differing phenomena, but are typically visualized using common geometric shapes.

### Catchment topology, comparison with the ISO topology model

The catchment model in this standard defines the common outlet to which all waters flow as the topological outfall of a corresponding catchment. With respect to the catchment, the topological outfall is defined as the inflow node of a receiving catchment or the outflow node of contributing catchment. The catchment forms the topological link connecting the outfall nodes located on the bounding catchment divide. Determined by the orientation of the outfall, a receiving catchment links inflow nodes to the outflow node downstream, and a contributing catchment links an outflow node to upstream inflow nodes. Repeating this topological pattern of connecting inflow and outflow nodes, a network of upper and lower catchments can be built, which can then be traced upstream or downstream. Depending on the perspective of a study, the network of upper and lower catchments is realized as hydrographic or channel network of seemingly connected watercourses, or as a network of logically connected stations. Each such catchment network view forms a hydrological system wherein the pattern of catchment and outfall (link and node) is realized as water bodies or channels and associated reference locations or stations. These realizations are represented in data products using geometric points, lines or polygons.

The topological concept of outfall node and catchment link applies to all defined special subtypes of a catchment. Specializations are defined with respect to (catchment) network connectivity. A dendritic catchment is defined, which is connected to other catchments through an identified outflow node which coincides with the inflow node of an associated lower catchment; an interior catchment is defined which is generally not connected to other catchments, but may temporarily participate in the network. With respect to the aggregation of catchments, a catchment aggregate is defined which encompasses dendritic catchments and envelopes interior catchments.

A general topology model of nodes and edges is defined in ISO 19107: Spatial Schema. While consistent with conceptual geometry, this topology model is defined separate from the geometric realization and may provide a general framework to define the scale-free catchment topology using the model of nodes and edges. Such a model can be seen as being comparable to the linear shape and origin of a river reference system as well as the connecting link and node to navigate in the catchment network upstream or downstream. A catchment may be topologically realized as a solid bounded by inflow/outflow faces, each face bounded by inflow/outflow edges, and each edge bounded by inflow/outflow nodes. Each of these realizations can be represented using an appropriate geometry type.

Understanding the catchment as a hydrologically determined complex, various multiple realizations of the logical catchment may be derived from the basic topological feature realization. The domain-specific feature types and properties defined in this standard to model the catchment’s topology, its feature realization(s) and geometric representation(s), may also be used to express catchment topology expressed in linked data and data products. Such data and products will normally represent a specific aspect of the hydrosphere or a particular place or time where water occurs or is distributed. Furthermore, special network or routing models may be related to the hierarchical network of catchments.

### River Reference System, comparison with ISO linear referencing model

It is common practice in hydrology to add new features (typically observation stations, but also designated reaches, or flood plain zones) to an existing network of such features, and to reference such features along a network of watercourses. Provided that the hierarchical network of catchments is realized, and that the realized catchments have linear realizations, any feature can be referenced along this linear realization of the catchment based on catchment topology alone. This is possible considering that any location on the land surface can be an outfall of a catchment. In this case, the new location is placed as an outfall in an existing catchment network relative to an already established outfall used as a referent. This concept of an indirect position requires a reference system ‘along a river’.

This standard defines a simple linear river reference system with flowpath realization of a catchment as its shape and the outfall of the catchment as its origin. Understanding the feature of interest as outfall of a corresponding catchment, the unknown position of this outfall can be determined relative to an outfall upstream or downstream. Each outfall is potentially the origin of its own river reference system and uses the flowpath realizing the catchment between this origin and another already established outfall (inflow or outflow node) as its shape. The position is provided as the distance (length) of the curve representing the flowpath between both outfalls, or as percentage of the entire distance. In order to use an interpolative (percentage along a flowpath) linear referencing method, the flowpath used as the river reference system's shape must be bounded by an inflow and outflow node, whereby one of these must be the origin.

A standard model for a linear referencing is described in ISO 19148: Linear Referencing.   It defines the position of an event, or referenced feature, located on a linear element (understood as directed edge in terms of the ISO 19107) as a distance relative to a referent using a particular referencing method. The indirect position concept defined in hydrologic feature standard is basically comparable to the general concept defined in ISO 19148. In the terms used by ISO 19148, the one-dimensional flowpath feature (topological realization of a catchment) can be understood as the linear element; the already established outfall can be thought of as the referent; and the newly referenced outfall as an event.

Using that the flowpath (feature) as the only one linear element forming the axis in the reference system always starts at the origin and is directed towards the referent, and that a measurable representation of the flowpath exists, the river reference system defined in this standard describes a very common method of referencing along a linear feature comparable with the core concepts of the ISO Linear Referencing model. It allows the positioning of a catchment’s outfall relative to an identified reference location without the complexity of the ISO model by ‘measuring’ the geometric representation (curve) of the linear flowpath realizing the catchment (idea) topologically.

It is one intended goal of this standard to provide a pragmatic approach to place a feature of interest on a river relative to an already existing feature located on this river using a simple linear referencing along the river under the requirement to use terms that are common in the hydrology domain, as well as to include a simple statement on precision and accuracy of the assigned position. Against this background, the concepts defined in the ISO 19148 are not ‘imported’, but may be implemented for more complex use cases using another type of catchment realization as shape. Specializing the CS_Linear CS (described in ISO 19111: Referencing by coordinates), the river reference system described in this standard does not require changes to the OpenGIS ® Abstract Specification, Part 19: Linear Referencing.

### Catchment Network Navigation, comparison with ISO network (navigation) model

Thinking about catchments realized as topological edges bounded by outfall nodes, a dendritic network of catchments may be traced upstream from the sea or a sink to the inflow/outflow nodes of (multiple) upper catchments. From there, one can navigate further ‘upstream’ eventually arriving at the outflow node of the headwater catchments. Starting at a spring, the catchment network can be traced in the ‘downstream’ direction, first to the single outflow node to which the catchment contributes then further downstream eventually arriving at the inflow node of a branching (non-dendritic), estuary or delta. In a given realization, a sequence of linear flowpaths, each realizing a catchment connected in the catchment network, may be drawn as streams or watercourses, which may or may not be geometrically connected in the representation. For example, water bodies and channel parts of a particular network may be displayed using different geometric shapes, and may look connected on a map even if they are not.

A standard model used for tracking and navigation, is defined in ISO 19133: Tracking and Navigation standard. It defines a network complex of junctions and links specializing the directed node and directed edge types defined in ISO topology model. Provided that a catchment is topologically realized comparable with the ISO topology model as topological edge (flowpath) and the outfall as a (inflow/outflow) node, the catchment network (as well as its hydrographic, geomorphologic or hydrometric realizations) can be navigated comparable with the concepts of the ISO network (navigation) model. With reference to catchment topology, this allows to navigate from a realized outfall to realized outfall along the flowpath realizing a catchment, from fixed landmark to fixed landmark along a water body, from section to section along a channel, or from station to station along a virtual line.

To provide a simple approach to navigate upstream or downstream in the hydrographic network of streams, the channel network of watercourses, or a logical network of  stations, using a terminology common in the hydrology domain is one intended goal of this standard. Against this background, the concepts defined in the ISO 19133 are not ‘imported’, but may be implemented for more complex use cases, e.g. network routing including interruptions and turns.

# Clause containing normative material

## The HY_Features conceptual model

This standard defines the HY_Features conceptual model as a standard for the identification and description of hydrologic features reflecting both hydrologic significance and topological connectivity of hydrologic features. HY_Features formalizes the fundamental relationships between components of the hydrosphere. It describes the hydrosphere as a hierarchical network of hydrologically connected catchments, their organization in networks of catchments and/or waterbodies, and their various visual and topological realizations.

Core concepts of HY_Features are: 1) an abstract idea of 'catchment' which has many possible 'realizations', 2) upstream - downstream catchment topology and nested hierarchy, 3) aggregation of networks of watercourses within catchment networks, and 4) linear referencing along a river using a nominal main flow path. The single concept that governs HY_Features is that any place on the land surface can be thought of as the outfall of a corresponding catchment and be placed into a hydrologic topology of connected, often named, hydrologic features.

The conceptual model is implemented in several discrete modules. It is intended that implementations of the conceptual model need to consider only those parts of the model required. An implementation may include or exclude feature properties, or allow cardinality of one or more associations to be ‘nillable’. Table 1 lists the application schemas, the leaf packages included and the concepts reflected therein.

Table 1: HY_Features modules, concepts reflected, and leaf packages included

| Application schema | Concepts reflected | Leaf packages included |
| --- | --- | --- |
| HY_HydroFeature | fundamental properties and relationships between features governed by the physical laws of Hydrology, naming of hydrologic features, location of hydrologic feature along a line | NamedFeature, HydroComplex, RiverPositioningSystem |
| HY_SurfaceHydroFeature | hydrologic features on the Earth’s land surface without complexity and detail of hydrologic and hydraulic models | ChannelNetwork, HydrographicNetwork, WaterBodyTypes, Storage |
| HY_HydrometricNetwork | hydrometric network of logically connected hydrometric features located on or along a hydrologic feature | --- |

The conceptual model is expressed in the Geographic Information Conceptual Schema Language (ISO 19103:2005) based on the Unified Modeling Language (UML). The organization into packages and package dependencies are shown in Figure 17. The following sections describe requirements classes for each application schema, whereby each feature addressed in the requirements shall be understood as an instance of the GF_FeatureType (aka FeatureType) «metaclass».

Figure 17: HY_Features modules and packages

## The HY_Features conceptual conformance (mapping)

The HY_Features model is a 'conceptual model', not intended to be directly implementable for data exchange or persistence. The conformance target of the HY_Features model is therefore a logical model of an implementation that encodes aspects of the HY_Features model.

Conformance to the HY_Features model is a matter of being able to unambiguously identify what elements of an implementation schema map to the HY_Features model, and inclusion of all mandatory properties of the defined Feature Types in such mappings.

The HY_Features conceptual model provides the basis for determining whether two references to hydrologic features are references to the same feature. i.e. to specify the real world type of features independent of their implementation or format. More specifically, it provides the means to distinguish between the reference concept (e.g. a catchment) and its realizations as geographic features (e.g. flowpath, catchment area or boundary), and hence to declare that different realizations share common hydrological connectivity.

Disparate systems describing hydrologic features may be mapped to the equivalent HY_Features definitions to disambiguate the local usage of terminology and specific implementation choices made.

Note that a direct encoding of HY_Features to an implementation format such as RDF may be implemented through annotation or direct correspondence of names to the HY_Features elements.

| Requirements Class | /req/hy_features_conceptual_model |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_Features Conceptual Conformance |
| Dependency | /iso/19109/ |
| Requirement | /req/ hy_features_conceptual_model/mapping |

An implementation schema conforming to HY_Features SHALL provide a formal mapping from one or more Feature Types present in the implementation schema to Feature Types defined in this standard specification, including all mandatory properties defined by the realized HY_Features concept. Default values to be assumed must be specified in this mapping.

## The HY_Features data conformance (encoding)

As a conceptual model HY_Features does not specify conformance requirements regarding the structure of possible encodings. It does however specify that equivalence of feature instances can be expressed. The requirement that arises is therefore that the content of feature identification elements can be matched among implementations. This does not demand the use of identical identifiers across different implementations, but it does require that implementations provide a mechanism to match identifiers from different schemes.

| Requirements Class | /req/hy_features_content |
| --- | --- |
| Target type | Dataset |
| Name | HY_Features Data Conformance |
| Dependency | /req/hy_features_conceptual_model |
| Dependency | /iso/19109/ |
| Dependency | /iso/19150/ |
| Dependency | /iso/19136/ |
| Requirement | /req/ hy_features_conceptual_model/identifiers |

Implementations of HY_Features SHALL either use common identifiers for instances of Feature Types mapped to the same underlying HY_Features Feature Type, OR provide a mechanism to match identifiers from different identification schemes.

## The Hydro Feature application schema

The Hydro Feature application schema provides the core concepts of a named hydrologic feature, of a catchment and its multiple realizations, and of a river positioning using a linear referencing. Hydrologic features are identified by hydrologically significant characteristics and feature topology according to hydrologic rules. Providing a standard terminology for the typical relationships between hydrologic features allows the hydrosphere to be expressed in a consistent way across multiple data products, regardless of various spatial or temporal representations.

Hydrologic features are usually named in cross-jurisdictional and multi-lingual contexts. The Hydro Feature schema provides a concept for a named hydrologic feature which allows the use of multiple names and identifiers without the need for a formal naming model. The named hydrologic feature is further described using various domain-specific feature types, which specify properties of the specializations to define one or more aspects of the hydrology phenomenon (Figure 18).

The Hydro Feature schema provides the core model of catchments and their multiple realizations. The catchment model denotes the hydrologic definition of a catchment by its outlet, conceptualized as HY_Outfall, and the interaction of catchments through such defined outfalls. This allows division of the hydrosphere into a logically connected network of catchments. Depending on the perspective of a particular study, the logical catchment can be realized in multiple ways by geometric or topologic features. The Hydro Feature schema provides a model to place a catchment's outfall relative to a feature which realizes the logical outfall. The river positioning model provides a river reference system which allows linear referencing of an outfall (typically a monitoring location) using the linear realization of a catchment that corresponds to another outfall (typically a confluence).

Figure 18: Hydrologic features describing separate aspects of the hydrology phenomenon

The definitions applied in the Hydro Feature schema are rooted in the definitions given in the WMO/UNESCO Glossary of Hydrology regardless of their application context in respect to the Earth's surface. For the purpose of testing the applicability of the conceptual model in the context of surface water hydrology, the definitions in this standard refer to surface water hydrology. A conceptual model capturing the specifics of features associated with the groundwater domain is developed with reference to the OGC WaterML 2: Part 4 – GroundwaterML2 standard [6].

The Hydro Feature schema contains the leaf packages: NamedFeature, HydroComplex, and RiverPositioningSystem. Figure 19 shows the external dependencies.

Figure 19: Hydro Feature – external dependencies

| Requirements Class | /req/hy_hydrofeature/* |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_HydroFeature (abstract) |
| Dependency | /iso/19103/ |
| Dependency | /iso/19107/ |
| Dependency | /iso/19111/ |
| Requirement | /req/hy_hydrofeature/namedfeature/* |
| Requirement | /req/hy_hydrofeature/hydrocomplex/* |
| Requirement | /req/hy_hydrofeature/positioning/* |

### The Named Feature model

The Named Feature model (Figure 20) denotes the abstraction of the hydrology phenomenon as a named hydrologic feature. It provides an approach to identify a named hydrologic feature in cross-jurisdiction and multi-lingual contexts by considering the cultural, political and historical aspects of names assigned to hydrologic features in common usage.

Figure 20: Named Feature (UML class diagram, /req/hy_hydrofeature/namedFeature/hydrofeature )

The HY_HydroFeature feature type defines the basic feature to reflect overall properties hydrologic features have, such as identifier and name. The HY_HydroFeature feature type is further specialized by separate feature types to reflect different aspects of hydrology by the typical characteristics each specialization may have. Each specialization inherits the properties from generalization; HY_HydroFeature type has one association: name.

The name associates a name given to the hydrologic feature in cultural, political or historical context. If required, this association shall be used where names are assigned to a feature instance in cross-jurisdictional and multi-lingual contexts, which may occur with trans-boundary features.

| Requirements Class | /req/hy_hydrofeature/namedFeature/hydrofeature |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_HydroFeature |
| Dependency | /req/hy_hydrofeature/namedFeature/hydrofeaturename |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.name |

The HY_HydroFeatureName feature type provides an abstract pattern to handle cultural, political and historical variability of names. This allows the assignment of a referencable name for all or part of a hydrologic feature without necessarily have a formal model for naming. HY_HydroFeatureName has five attributes: name, namesPart, preferredBy, usage and variantSpelling. If required, an implementation shall use this type to describe the usage of multiple names. The usage type may be identified using the HY_NameUsage code list described in Annex B.4.

| Requirements Class | /req/hy_hydrofeature/namedFeature/hydrofeaturename |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_HydroFeatureName |
| Dependency | /iso/19103/ |
| Dependency | /iso/19115/ |
| Dependency | /req/hy_hydrofeature/namedFeature/nameusage |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.name |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.namespart |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.preferredBy |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.usage |
| Requirement | /req/hy_hydrofeature/namedFeature/hydrofeature.variantspelling |

### The Hydro Complex model

The Hydro Complex model conceptualizes the hydrologic definition of a catchment through an 'outfall' feature with the role of getting flow from a contributing catchment, or providing inflow to a receiving catchment (Figure 21 and Figure 22). Conceptually, each catchment has an outfall, and any outfall has a corresponding catchment, even if catchment and/or outfall may not be present in a particular application. A catchment interacts with upper and lower catchments via associated outfalls, and ultimately contributes flow to the outfall of a containing catchment. The catchment should be understood as the logical link between two outfalls.

The Hydro Complex model implies a collection of hydrologic features that form a hydrologically closed system. The union of a catchment, its inflow and outflow (conceptualized as outfalls) is realized by typical hydrologic features to form a single hydrologic complex. A topological realization of the logical catchment is always of higher topological dimension than the realization of the corresponding outfall in terms of a topological boundary. For example, a linear flowpath realizing a catchment may be understood as an edge between inflow and outflow nodes; the areal realization of a catchment as a face bounded by linear inflow and outflow.

The Hydro Complex model allows the realization of a logical outfall to be any arbitrary location with various realizations of its own. Such a 'real' outfall provides an identifiable reference feature to which alternative catchment realizations may refer. This very general outfall feature supports establishment of topological relationships between hydrographic features, or between hydrographic and hydrometric features. The Hydro Complex model also allows for catchments to be recognized through reference to an outfall even if stream networks, catchment areas or catchment boundary are not available. It is intended that hydrological reporting applications may use this model without the full complexity and detail of scientific catchment models.

#### Catchment

The HY_Catchment feature type represents a hydrologically determined feature through association with an outfall, and the logical network of catchments that forms a connected network of smaller catchments and outfalls within the catchment. Each HY_Catchment may provide associations between its many realizations within a hydrologically determined hydrologic complex. These realizations include both geometric and topological realizations such as an edge 'bounded' by inflow and outflow nodes. HY_Catchment is an abstract class and may be further specialized with respect to catchment interaction.

Figure 21: Catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment )

The HY_Catchment type (Figure 21) specializes the general HY_HydroFeature class. Through generalization, HY_Catchment inherits the name property, and carries the code attribute and the associations: outflow, inflow, containingCatchment, containedCatchment, conjointCatchment, upperCatchment, lowerCatchment, realization.

The code attribute may be used to assign a unique identifier to the catchment in given context. If required, the code attribute shall be implemented using a controlled classification or coding system. Example: WMO Basin Codes.

The outflow and inflow associations describe the outfall in terms of outflow or inflow to or from the corresponding catchment. Assuming a dendritic network of catchments, the outflow of a contributing catchment coincides with the inflow to a receiving catchment. This allows description of upstream-downstream relations. If required, this association shall be used to identify the place to which flow is contributed, or where flow is received from.

Figure 22: Catchment and outfall (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.inflow , /req/hy_hydrofeature/hydrocomplex/catchment.outflow )

The containingCatchment and containedCatchment associations connect the nesting of catchments in a simple “is-in” containment hierarchy as typically used for high-order organization of management and reporting units. If required, this association shall be used to identify a nesting catchment or the catchments nested therein.

Figure 23: Containing / contained catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.containingcatchment , /req/hy_hydrofeature/hydrocomplex/catchment.containedcatchment )

The conjointCatchment association describes the interaction of a catchment with another catchment crossing an internal boundary line. This line may be a divide separating adjacent catchments, or a diffuse divide between non-delineated sub-catchments within an encompassing catchment, or a fictive line between distant catchments. If required, this association shall be used to identify a catchment contributing with others to a 'joined' outfall. Assuming a dendritic network of catchments, where each catchment is determined by its single outflow, this association may be used to summarize diffuse inflow into an encompassing catchment, as required to describe inflow to headwater catchments.

Figure 24: Conjoint catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.conjointcatchment )

The upperCatchment and lowerCatchment associations connect the catchment to the adjacent catchment above or below. This allows the description of connected catchments without knowing their inflow or outflow outfalls. If required, this association shall be used to trace the catchment network in an upstream direction from mouth to source, or downstream from source to mouth.

Figure 25: Upper / lower catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchment.uppercatchment, /req/hy_hydrofeature/hydrocomplex/catchment.lowercatchment )

The realization association relates the catchment to a feature which realizes the logical catchment. This supports linking multiple realizations of the same catchment. If required, this association shall be used identify a particular realization. In case of a topological realization, the realization of the catchment shall be of higher dimension than the realization of the outfall.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/catchment |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_Catchment |
| Dependency | /iso/19103/ |
| Dependency | /req/hy_hydrofeature/namedFeature/hydrofeature |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfall |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.outflow |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.inflow |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.containingcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.containedcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.conjointcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.uppercatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.lowercatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchment.realization |

#### Catchment Aggregate

The HY_CatchmentAggregate feature type (Figure 26) specializes the HY_Catchment as a set of non-overlapping dendritic and interior catchments arranged in an encompassing catchment. This allows description of multiple inflows into a catchment aggregate through several hydrologically discrete sub-catchments each with a single inflow, and contributing to a joined outflow of the catchment aggregate, including the 'nillable' outflow of interior catchments. Nillable is meant to signify that the outfall exists, in the form of flow to the subsurface or atmosphere but is unknown in a given implementation. The catchment aggregate may be part of a containing catchment at the next higher level of hierarchy, which consists of similar-scale neighboring catchments. The catchment aggregate does not necessarily imply a series of nested containing catchments. It primarily allows navigation to the 'highest' level system (total drainage basin) as typically used for reporting purposes.

Figure 26: Catchment aggregate (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchmentaggregate )

HY_CatchmentAggregate inherits from generalization the outflow, inflow, containingCatchment, containedCatchment, conjointCatchment, upperCatchment, lowerCatchment, and realization properties, and associates the exorheicDrainage and endorheicDrainage.

The exorheicDrainage association references an exorheic drained catchment connected to others in a dendritic network. The endorheicDrainage association references an endorheic drained catchment, temporarily connected to the enveloping aggregate. If required, these associations shall be used to identify aggregated catchment parts which permanently or temporarily interact with other catchment parts at the same hierarchy level.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/catchmentaggregate |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_CatchmentAggregate |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/dendriticcatchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/interiorcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentaggregate.exorheicdrainage |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentaggregate.endorheicdrainage |

#### Dendritic Catchment

The HY_DendriticCatchment feature type (Figure 27) specializes the general HY_Catchment class as a catchment which is determined by a single common downstream catchment. It represents the catchment as the topological link between an inflow and an outflow. This allows catchments to be connected in a dendritic network by upstream-downstream relationships without knowing the complex hydrology between inflow and outflow. This concept requires a stable identifier purposefully assigned to the catchment and that catchments are delineated as a simple tree hierarchy. The dendritic nature of this class is enforced through constraints that the catchment must have one and only one outflow (which can be unknown), must have one and only one lowerCatchment (which may be unknown), and must have an outflow that contributes to only one receiving catchment.

Figure 27: Dendritic catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/dendriticcatchment )

HY_DendriticCatchment inherits from generalization the code, outflow, inflow, containingCatchment, containedCatchment, conjointCatchment, upperCatchment, lowerCatchment, and realization properties, and associates with the encompassingCatchment.

The encompassingCatchment association relates to the dendritic catchment the aggregate encompassing the catchment. If required, this association shall be used to identify the catchment encompassing one or more exorheic or endorheic drained catchments contributing flow to the common outlet, either from a single identified inflow, or in joining with other sub-catchments crossing a divide internal to the encompassing aggregate catchment.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/dendriticcatchment |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_DendriticCatchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentaggregate |
| Requirement | /req/hy_hydrofeature/hydrocomplex/dendriticcatchment.encompassingcatchment |

#### Interior Catchment

The HY_InteriorCatchment feature type (Figure 28) specializes the general HY_Catchment class as a catchment which is generally not connected to other catchments. This class describes the interior catchment as a catchment enveloped by other catchments to which it may temporarily contribute. While the interior catchment concept precludes flow to neighboring surface catchments, holistically it is a candidate for establishing connections to groundwater or atmospheric systems.

Figure 28: Interior catchment (UML class diagram, /req/hy_hydrofeature/hydrocomplex/interiorcatchment )

HY_InteriorCatchment inherits from generalization the code, outflow, inflow, containingCatchment, containedCatchment, conjointCatchment, upperCatchment, lowerCatchment, and realization properties, and associates the envelopingCatchment.

The envelopingCatchment association relates to the interior catchment the aggregate surrounding the catchment. If required, this association shall be used to identify the catchment enveloping one or more endorheic drained catchments contributing 'nillable' flow to the common outlet.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/interiorcatchment |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_InteriorCatchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentaggregate |
| Requirement | /req/hy_hydrofeature/hydrocomplex/interiorcatchment.envelopingcatchment |

#### Outfall

The HY_Outfall feature type (Figure 29) conceptualizes the hydrologically determined outfall of a corresponding catchment (Figure 22). The logical outfall represents the logical place where a catchment interacts with another catchment, i.e. where the outflow of a contributing catchment becomes inflow into a receiving catchment. A catchment may receive flow from several catchments or contribute flow to several catchments through a single logical outfall. Logically placed in reference to a catchment which links inflow and outflow, an outfall has a position relative to another outfall that is 'fixed' by the catchment. Through unique identity, each outfall feature may associate different outfall realizations within an hydrologic complex given that each realization has the same hydrologic determination. This includes the topological realization as a node in terms of the 'boundary' on a catchment edge.

Figure 29: Outfall (UML class diagram, req/hy_hydrofeature/hydrocomplex/outfall )

HY_Outfall carries the associations: contributingCatchment, receivingCatchment, relativePosition and outfallRealization.  An outflow-on-flowpath and an inflow-on-flowpath constraint are defined such that whenever the position of an outflow, or inflow, is determined using a flowpath shape, the corresponding catchment shall be realized as HY_Flowpath feature type.

The contributingCatchment association relates the outfall to the catchment that contributes flow to it. This allows relation of a catchment's outflow to an identified inflow and to determine its position through referencing an outfall. If required, this association shall be used to identify the catchment connecting the outfall and an inflow used for reference.

The receivingCatchment association relates to the outfall to a catchment that receives flow from it. This allows relation of a catchment's inflow to an identified outflow and to determine its position through referencing an outfall. If required, this association shall be used to identify the catchment connecting the outfall and the outflow used for reference.

The relativePosition association assigns a position to the outfall relative to a reference location (typically an outfall realization) fixed in the logical network of catchments. This means that the position of an outfall is determined relative to another realized outfall. If required, this association shall be used to assign a position to an inflow or outflow of a catchment using existing reference location(s). Commonly, this is used to locate an outfall such as at a hydrometric station, to a reference outfall such as a confluence but it can be used to locate any outfall relative to another.

The outfallRealization association relates the outfall to a feature which realizes the logical outfall. If required, this association shall be used to describe the 'real' object considered to be outfall of a catchment. In case of a topological realization, the realization of the outfall shall be of lower dimension than the realization of the corresponding catchment.

| Requirements Class | req/hy_hydrofeature/hydrocomplex/outfall |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_Outfall |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Dependency | /req/hy_hydrofeature/hydrocomplex/indirectposition |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfall.contributingcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfall.receivingcatchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfall.relativeposition |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfall.outfallrealization |

#### Catchment Realization

The HY_CatchmentRealization feature type (Figure 30) conceptualizes the multiple realization of an 'un-realized', logical catchment, by typical features used to describe a catchment as a unit of study shared across sub-domains and studies. HY_CatchmentArea, HY_CatchmentBoundary, HY_Flowpath and HY_WaterEdge are special types of catchment realization defined to topologically realize the hydrologically determined logical catchment in terms of face, edge, and node, as well as to reflect the connectivity of catchments by hydrologic features connected in networks. Topologically, the catchment area is a face bounded by catchment boundary and flowpath. Flowpath and water edge are topological edges each bounded by an inflow node and an outflow node. The HY_HydroNetwork type realizes a logical catchment as a collection of connected network features, the HY_CartographicRealization as a set of map layers.

The catchment realization concept implies a hydrologic complex in that, if a catchment realization exists, they are in the same hydrologic complex as the catchment they realize. In this way, any feature realization of a logical catchment has the same hydrologic determination as the realized catchment. If the realized catchment is connected with other catchments via its outfall, possible feature realizations are also connected.

Figure 30: Catchment realization and its specialization (UML class diagram, /req/hy_hydrofeature/hydrocomplex/catchmentrealisation)

The catchment realization feature types defined in this standard refer to objects on the land surface for the purpose of surface water hydrology. In other contexts, other types of catchment realization may exist. Catchment realizations that do not conform to those defined in this standard, for instance realizations in 3 or 4 dimensional perspectives, may use the general HY_CatchmentRealization type.

HY_CatchmentRealization carries the realizedCatchment association implying the hydroComplex. ‘Tagged values’ are used to describe the hydroComplex which encompasses a realization feature in the same hydrologic complex as the realized catchment.

The realizedCatchment association relates a particular realization with the catchment it realizes. Referencing the hydrologic complex encompassing the catchment and its realizations, allows a catchment's existence to be recognized and linked to multiple realizations without the complexity and full detail of a scientific model. By referencing the fundamental hydrologically determined catchment, topological relationships can be established and common identifiers assigned. If required, this association shall be used to identify the unit of study realized in a domain-specific feature.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_CatchmentRealization |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentrealization.realizedcatchment |

The HY_Flowpath feature type specializes HY_CatchmentRealization with respect to an implied linear geometric representation including a straight or curved line. Hydrologically, the flowpath is a line describing a moving particle of water. Topologically, the flowpath connects the inflow and outflow of the logical catchment, and is understood as an edge bounded by an inflow node and an outflow node, and corresponding to left-bank and right-bank catchment faces. The 'boundary' and 'spoke' properties are described by means of 'tagged values': the topological 'boundary' is of type HY_OutfallRealization, the topological 'spoke' of type HY_CatchmentArea.

Through generalization, HY_Flowpath inherits the realizedCatchment association including hydroComplex, and carries the properties: shape, and flowpathNetwork.

The shape attribute defines the linear geometric representation. If required, an implementation shall use a geometry type defined in ISO 19107: Spatial Schema, e.g. GM_Curve type.

The flowpathNetwork association defines a network as sequence of connected flowpaths. This concept requires a non-branching 'main-stem' of watercourses, and a single linear representation of each of these. If required, this association may be used to identify the network which realizes as a whole the catchment that contains the catchment realized by the single part.

| Requirements Class | req/hy_hydrofeature/hydrocomplex/flowpath |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_Flowpath |
| Dependency | /iso/19107/ |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Requirement | /req/hy_hydrofeature/hydrocomplex/flowpath.shape |
| Requirement | /req/hy_hydrofeature/hydrocomplex/flowpath.flowpathnetwork |

The HY_WaterEdge feature type specializes HY_CatchmentRealization with respect to an implied linear geometric representation including a closed line. Hydrologically, the water edge follows the line of intersection between a water body stratum and the confining depression, or channel. Topologically, the water edge realizes the catchment as an edge connecting inflow and outflow realized as nodes fixed at the inflow node and outflow node of the corresponding flowpath. The 'boundary' of type HY_OutfallRealization is described by means of 'tagged values'.

Through generalization, HY_WaterEdge inherits the realizedCatchment association including hydroComplex, and carries the properties: shape, waterEdgeNetwork.

The shape attribute defines the linear geometric representation. If required, an implementation shall use a geometry type defined in ISO 19107: Spatial Schema, e.g. GM_Curve type.

The waterEdgeNetwork association defines an aggregate of water edges forming a connected network. This concept requires an aggregate of non-overlaying edges, and a single linear representation of each of these. If required, this association may be used to identify the network which realizes as a whole the catchment that contains the catchment realized by the single part.

| Requirements Class | req/hy_hydrofeature/hydrocomplex/wateredge |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_WaterEdge |
| Dependency | /iso/19107/ |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Requirement | /req/hy_hydrofeature/hydrocomplex/wateredge.shape |
| Requirement | /req/hy_hydrofeature/hydrocomplex/wateredge.wateredgenetwork |

The HY_CatchmentBoundary feature type specializes HY_CatchmentRealization with respect to an implied linear geometric representation of connected curves, including a single polygon. The catchment boundary connect the inflow and outflow of the logical catchment, whereby inflow and outflow may overlay. It is topologically understood as an edge bounded by inflow node and outflow nodes, and corresponding to left-bank and right-bank catchment faces inside of the boundary. The 'boundary' and 'spoke' properties are described by means of 'tagged values': the topological 'boundary' is of type HY_OutfallRealization; the topological 'spoke' is of type HY_CatchmentArea.

HY_CatchmentBoundary inherits from generalization the realizedCatchment association incl. hydroComplex, and carries the properties: shape, and boundaryNetwork.

The shape attribute defines the linear geometric representation. If required, an implementation shall use a geometry type defined in ISO 19107: Spatial Schema, e.g. GM_CompositeCurve type.

The boundaryNetwork association defines a network as mesh of connected boundaries. This concept requires a mesh of non-overlapping boundary lines, and a single linear representations of each of these. If required, this association may be used to identify the network which realizes as a whole the catchment that contains the catchment realized by the single part.

| Requirements Class | req/hy_hydrofeature/hydrocomplex/catchmentboundary |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_CatchmentBoundary |
| Dependency | /iso/19107/ |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentboundary.shape |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentboundary.boundarynetwork |

The HY_CatchmentArea feature type specializes HY_CatchmentRealization with respect to an implied areal geometric representation including a plane surface. The catchment area connecting the inflow and outflow of the logical catchment, is topologically a face bounded inwards by an inflow edge and outwards by an outflow edge. The 'boundary' is described by means of 'tagged values': the inward directed 'boundary' is of type HY_CatchmentBoundary, the outward directed 'boundary' of type HY_Flowpath. A topological 'spoke' is not defined in this standard.

HY_CatchmentArea inherits from generalization the realizedCatchment association incl. hydroComplex, and carries the properties: shape, and areaNetwork.

The shape attribute defines the linear geometric representation. If required, an implementation shall use a geometry type defined in ISO 19107: Spatial Schema, e.g. GM_Surface type.

The areaNetwork association defines an aggregate of catchment areas forming a connected network. This concept requires a non-overlapping aggregate of areas, and a single areal representation of each of these. If required, this association may be used to identify the network which realizes as a whole the catchment that contains the catchment realized by the single part.

| Requirements Class | req/hy_hydrofeature/hydrocomplex/catchmentarea |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_CatchmentArea |
| Dependency | /iso/19107/ |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchmentrealization |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentarea.shape |
| Requirement | /req/hy_hydrofeature/hydrocomplex/catchmentarea.boundarynetwork |

The HY_HydroNetwork feature type specializes HY_CatchmentRealization with respect to a network of connected hydrologic features. Such a network realizes in the hierarchical network of logically connected catchments contained in a higher-order catchment. It may be a sequence of flowpaths, an aggregate of water edges, an aggregate of catchment areas or a mesh of catchment boundaries. HY_HydroNetwork inherits the realizedCatchment association including hydroComplex, and carries the associations flowpath, waterEdge, catchmentBoundary and catchmentArea.

The HY_CartographicRealization feature type specializes HY_CatchmentRealization separate cartographic layers or maps, displaying a network of hydrologic features which may be connected at the representation level, or not. HY_HydroNetwork inherits from generalization the realizedCatchment association including hydroComplex.

#### Outfall Realization

The HY_OutfallRealization feature type (Figure 31) conceptualizes the idea that the an outfall can be realized by practically any feature of interest. Using the outfall to define and reference the hydrologic determination of a catchment, hydrologic features may be associated to a corresponding catchment through reference to the feature that realizes its outfall. Any feature that can be identified as (said to be) the outfall of a catchment may realize the logical outfall. Typically this will be a permanent, stable location that is fixed and/or referenced by coordinates.

Landmarks such as confluences, points corresponding to vertical sections, or the position of a monitoring station on a river are typical outfall realizations. In other than surface water contexts other types may realize a catchment's logical outfall. Other kinds of outfall realizations that don't carry normal surface water characteristics as defined in this standard, e.g. a spring where groundwater enters the surface, an arbitrary point projected onto the surface, or an outfall that collects many disjoint locations may use or specialize the general HY_OutfallRealization type.

Topologically, the HY_OutfallRealization should be understood to be the boundary of the corresponding catchment, and always of lower topological dimension than the catchment. With respect to the typical topological realization of a catchment as an edge, a 'spoke' property of type HY_Flowpath is described by means of a 'tagged value' and used to associate an outfall realization with its upstream and downstream flowpaths. Even though the topological realization of an outfall is typically as a node between catchment edges, an outfall realization may also have any geometric representation, including a single point.

Figure 31: Outfall Realization (UML class diagram, /req/hy_hydrofeature/hydrocomplex/outfallrealization )

HY_OutfallRealization carries the realizedOutfall association implying the hydroComplex feature collection, and carries two attributes: shape and realizedOutfallType.

The realizedOutfall association identifies the catchment that contributes to the outfall, or that receives flow from the outfall. This allows identifiers to be assigned to a catchment even if the flowpath, catchment area or stream network are not known or available. If required, this association shall be used to identify the logical outfall which is referenced by the hydrologic feature that realizes the logical catchment.

The shape attribute defines the geometric representation of the realized outfall with the option to use a geometry type defined in ISO 19107: Spatial Schema, if required.

The realizedOutfallType attribute provides a list of terms in common use to express the type of the realized outfall. If required, an implementation may use a term from the HY_RealizedOutfallType code list. Note that alternative code lists may be used but should be related to the terms in Annex B.1 using an appropriate formalism.

| Requirements Class | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| --- | --- |
| Target type | Implementation schema |
| Name | HY_CatchmentRealization |
| Dependency | /iso/19107/... |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfall |
| Dependency | /req/hy_hydrofeature/hydrocomplex/typeofoutfall |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfallrealization.realizedoutfall ] |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfallrealization.shape ] |
| Requirement | /req/hy_hydrofeature/hydrocomplex/outfallrealization.realizedoutfalltype ] |

### The River Positioning System model

The River Positioning System (Figure 32) provides a simple model to place a feature of interest 'on a river' using its topological realization. It introduces the concept of indirect position where a position is determined relative to an already established reference location. This concept uses a linear river reference system whose origin is set at the outfall of a catchment, and whose linear shape is given by a flowpath realizing the catchment between the origin (outfall) and the reference location or another outfall. It is important to note, that each logical catchment has its own reference system, and must have one outfall (origin) and one linear flowpath realization (shape).

The River Positioning System references the topological realization of catchment and outfall within an implied hydrologic complex in such that the origin and referent of the reference system are nodes on the boundary of the flowpath shape. Given that a flowpath realizes a logical catchment between inflow and outflow nodes, the feature of interest being referenced realizes an inflow or outflow node of a catchment that is determined by the corresponding reference location upstream or downstream of an origin outfall. Using reference locations in both directions allows placement of a feature of interest at an interpolated distance between the identified inflow and outflow nodes on a flowpath even if the realized catchment is not explicitly delineated.

Figure 32: River Positioning System (UML class diagram, /req/hy_hydrofeature/positioning/indirectposition, /req/hy_positioning/riverreferencesystem )

The HY_IndirectPosition feature type defines the indirect position, expressed either as the absolute distance along a flowpath to a reference point or the relative distance (percent) along a flowpath (termed interpolative). Indirect position assigns a position to a feature of interest (realizing an outfall) in reference to an existing outfall realization (or realizations for an interpolative position) using a linear river reference system. HY_IndirectPosition carries four properties: absolute, relative, referenceLocation, riverReferenceSystem. An implementation may use ISO 19148: LinearReferencing with the feature to be placed as an ‘Event’ at a locating indirect position in distance to an already located outfall realization, and this distance expressed as a linearElement (flowpath) fromReferent (origin outfall realization) towardsReferent (bounding outfall realization).

The absolute attribute provides a distance expressed as an absolute value, including an indication of accuracy and precision of the absolute value; the relative attribute provides an expression for the relative interpolative distance value. If required, an implementation shall use these attributes to express the absolute or relative value using the HY_DistanceToRefPoint and HY_RelativePosition data types, or other basic types defined in ISO 19103: Conceptual Schema. To express a relative position verbally, a term from the HY_RelativePositionDescription code list of terms commonly used in hydrology to describe a spatial relationship between two locations may be used. Note that alternative code lists may be used but should be related to the terms in Annex B.2 using an appropriate formalism.

The referenceLocation association describes an existing outfall realization used for reference. If required, this association shall be used to identify a permanent reference location relative to which a position is assigned to an outfall, or the feature of interest realizing the outfall.

The riverReferenceSystem association describes the special linear coordinate system applied to assign a position. If required, this association shall be used to identify the origin and shape of a reference system used to locate an outfall or the feature of interest realizing the outfall relative to others.

| Requirements Class | /req/hy_hydrofeature/positioning/indirectposition |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_IndirectPosition |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| Dependency | /req/hy_hydrofeature/positioning/distancetorefpoint |
| Dependency | /req/hy_hydrofeature/positioning/relativeposition |
| Dependency | /req/hy_hydrofeature/positioning/riverreferencesystem |
| Requirement | /req/hy_hydrofeature/positioning/indirectposition.absolut |
| Requirement | /req/hy_hydrofeature/positioning/indirectposition.relative |
| Requirement | /req/hy_hydrofeature/positioning/indirectposition.referencelocation |
| Requirement | /req/hy_hydrofeature/positioning/indirectposition.riverreferencesystem |

The HY_RiverReferenceSystem feature class specializes the ISO LinearCS feature type for a linear coordinate system using inflow and outflow nodes on the linear flowpath. The origin of the river reference system is set by the located start of the flowpath realizing the catchment between the origin and the reference location. Provided that the located start of the flowpath is at the outfall of the catchment corresponding to the feature to be placed and the end is at an already located outfall upstream or downstream, the position on the flowpath is provided as the distance between its start and end. Alternatively, the position of a feature of interest can be determined as a percentage of the distance between two already located outfalls bounding the flowpath in upstream and downstream direction, whereby the start is located at one of these and the flowpath is directed to the other already located outfall upstream or downstream of the start.

HY_RiverReferenceSystem inherits the axis property, and carries the associations linearElement and locatedStart.  Constraints  start-at-outflow  and start-at-inflow are defined such whenever the linear element (flowpath) starts at the outflow, or at the inflow, of the realized catchment, this located start is of type HY_Outfall.

The linearElement association defines the flowpath as the linear shape applied in the river reference system, and the locatedStart association defines the outfall as the origin. Bounded by inflow node and outflow node, the flowpath starts at the feature of interest realizing the origin. If required, these associations shall be used to identify the flowpath used as shape, and the start of the flowpath at a realized origin.

| Requirements Class | /req/hy_positioning/riverreferencesystem |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_RiverReferenceSystem |
| Dependency | /iso/19111/... |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfall |
| Dependency | /req/hy_hydrofeature/hydrocomplex/flowpath |
| Requirement | /req/hy_hydrofeature/positioning/riverreferencesystem.locatedstart |
| Requirement | /req/hy_hydrofeature/positioning/riverreferencesystem.linearelement |

## The Surface Hydro Feature application schema

The Surface Hydro Feature application schema provides common concepts of hydrologic features occurring on the land surface and specifies the core concepts defined in the abstract Hydro Feature application schema. This will enable contextually linked information models to build relationships between multiple realizations of the same catchments. Typical realizations of the catchment concept and logical outfalls can be described in a consistent way using standard terminology for the relationships between surface water features defined in this application schema.

The Surface Hydro Feature schema conceptualizes the accumulation of water on the land surface in water bodies, each made unique by its origin, size, or movement. With respect to the management and storage of water resources, a concept of water storage is provided and allows any water body type to be considered a managed reservoir.

Relying on a conceptual separation of water body and container, the Surface Hydro Feature schema defines a network of potentially connected depressions and channels on the land surface which periodically or continuously contain water. Separate from the hydrographic network of permanent or temporary water bodies, the channel network can be used as the connecting system.

The definitions in this schema are rooted in the definitions given in the WMO/UNESCO Glossary of Hydrology which defines a network of watercourse regardless of the location in respect to the Earth's surface. The conceptual model defined here has been vetted in the context of surface water hydrology. In other words, in this standard, 'channel network' and 'hydrographic network' refer to surface channels or other containers for surface-water bodies.

The Surface Hydro Feature application schema contains the leaf packages: Channel Network, Hydrographic Network, Water Body Types and Storage. Figure 32 shows the external and internal dependencies.

Figure 33: Surface Hydro Feature -  dependencies/req/hy_surfacehydrofeature/storage/*

### The Channel Network model

The Channel Network model defines a network of connected depressions and channels which, in its entirety, can realize a logical catchment (Figure 34). Usually this is a network of linear flowpaths and/or water edges realizing catchments connected to each other within the containing catchment realized by the entire network. This allows representation of the drainage pattern even if logically connected features may or may not be connected in the context of a particular representation.

Figure 34: Channel Network realizing the catchment (UML class diagram, /req/hy_surfacehydrofeature/channelnetwork/channelnetwork )

The channel network is defined independent of the hydrographic network. This separates the concerns of hydraulics, focused on the analysis and design of channels and conduits, from the concerns of hydrology, focused on the occurrence and movement of water over land and in water bodies. It allows a logical catchment to be realized as a network of connected channels and depressions, regardless of water is contained therein or not.

A single depression or channel may realize the logical catchment, either as part of the network (Figure 34) or via a reference feature which realizes the conceptual outfall of the catchment (Figure 35). For example, a point at an associated cross or longitudinal section may be considered to realize the outflow of the catchment which is realized by the channel expressed as a flowpath.

Figure 35: Depression and Channel realizing the outfall (UML class diagram, /req/hy_surfacehydrofeature/channelnetwork/depression, /req/hy_surfacehydrofeature/channelnetwork/channel )

#### Channel Network

The HY_ChannelNetwork feature type specializes the HY_HydroNetwork realization defined in the core model, specifically as an aggregate of surface depressions and surface channels which continuously or periodically contain water, without imposing a particular drainage pattern. This allows to represent the network, even if logically connected features may or may not be connected at the representation level. If the realized catchment is connected with other catchments via outfall, the channel network is considered connected to the channel network realizing these catchments. If required, an application focused on the structures containing a water body may use the defined relationships s to describe the realization of a catchment by the channel network, or network parts associated with the hydrographic network.

HY_ChannelNetwork associates the surfaceDepression and surfaceChannel, and carries a drainagePattern attribute; it inherits from generalization the realizedCatchment, flowpath and waterEdge, as well as catchmentBoundary and catchmentArea associations. Depending on the application, the channel network and the related features may be described by suitable attributes. A channel-path constraint is defined such that whenever the channel network is a network of flowpaths, a depression or channel is of type HY_Flowpath, and a channel-edge constraint such that depression or channel is of type HY_WaterEdge whenever the channel network is a network of water edges.

The drainagePattern attribute describes in general the drainage pattern. If required, an implementation may use a term from the HY_DrainagePattern code list. Note that alternative code lists may be used but should be related to the terms in Annex B.3 using an appropriate formalism.

The surfaceDepression association relates a depression which may contain stagnant water or not to the channel network. If required, this association shall be used to identify a depression which realizes the logical catchment either separately, or as part of the channel network.

The surfaceChannel association relates a channel which may contain moving water or not to the channel network. If required, this association shall be used to identify a surface channel which realizes the logical catchment either separately, or as part of the channel network.

| Requirements Class | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_ChannelNetwork |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/drainagepattern |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/depression |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/channel |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork.drainagepattern |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork.surfacedepression |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork.surfacechannel |

#### Depression

The HY_Depression feature type specializes the general HY_HydroFeature class. It describes land lower than the surrounding land as container for standing water. A depression is part of the network of channels and depressions forming the connecting system for the wherein water bodies stand as parts of hydrographic network. HY_Depression inherits from generalization the name property, and the channel-path and channel-edge constraints. It associates the surfaceNetwork in which it participates, a body of standingWater and a confinedWaterBody. A depression-edge constraint is defined such that whenever the depression is part of the surface network, the confined water body is of type HY_WaterEdge. An outflow-at-benchmark constraint is defined such that the outflow of the catchment which is realized by the channel network is of type HY_OutfallRealization whenever an outfall is realized on a water body stratum.

The standingWater association relates a body of stagnant water to a containing pool. This allows to establish a relationship to a water body carrying a permanent reference location which realizes the outfall of the catchment realized by the associated water body. If required, this association shall be used to identify the water body contained in the depression.

The confinedWaterBody association relates a stratum of the contained water body to the depression. If required, this association shall be used to identify the horizontal layer carrying a permanent reference location which realizes the outfall of the catchment which is realized by the depression.

| Requirements Class | /req/hy_surfacehydrofeature/channelnetwork/depression |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_Depression |
| Dependency | /req/hy_hydrofeature/namedfeature/hydrofeature |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbodystratum |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/depression.surfacenetwork |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/depression.standingwater |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/depression.confinedwaterbody |

#### Channel

The HY_Channel feature type specializes the general HY_HydroFeature class with respect to a natural or man-made, open or closed channel through or along which water may flow, or not. A channel is part of the network of channels and depressions which form the connecting system for the hydrographic network; a channel may have vertical sections at right angles to the main (average) direction of flow or along its centreline.

HY_Channel associates the channelNetwork in which it participates. It carries the associations: stream, bedProfileTransversal and bedProfileLongitudinal. HY_Channel inherits from generalization the confinedWaterBody association and the constraints depression-edge and outflow-at-benchmark. An outflow-at-profile constraint is defined such that whenever an outfall is realized at a bed profile, the outflow of the catchment realized by the channel network is of type HY_OutfallRealization.

The stream association relates to the channel a water body periodically or continuously flowing in the channel. This allows to establish a relationship to a water body carrying a permanent reference location which realizes the outfall of the catchment realized by the water body. If required, this association shall be used to identify the water body contained in the channel.

The bedProfileTransversal and bedProfileLongitudinal associations relate to the channel a transversal or longitudinal vertical shape. If required, this association shall be used to identify the vertical section carrying a permanent reference location which realizes the outfall of the catchment which is realized by the channel.

| Requirements Class | /req/hy_surfacehydrofeature/channelnetwork/channel |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_Channel |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/channelnetwork |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/depression |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/crosssection |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/longitudinalsection |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channel.channelnetwork |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channel.stream |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channel.bedprofiletransversal |
| Requirement | /req/hy_surfacehydrofeature/channelnetwork/channel.bedprofilelongitudinal |

### The Hydrographic Network model

The Hydrographic Network model defines a logical network of water bodies which in its entirety realizes a logical catchment (Figure 36), usually as a network of linear flowpaths and/or water edges realizing catchments connected to each other within a containing catchment that is realized by the entire network. This allows to represent the network of ‘blue lines’, even if logically connected features may or may not be connected at the representation level.

Figure 36: Hydrographic Network realizing the catchment (UML class diagram, /req/hy_surfacehydro/hydrographicnetwork/hydrographicnetwork )

The hydrographic network is defined independent of the channel network. This conceptual separation references to the specific concerns of hydrology studying the occurrence, accumulation, and circulation of water. It and allows to realize the logical catchment as a network of moving or standing water bodies, regardless of the channel wherein it may move, or not.

A single water body realizes the logical catchment either as part of the hydrographic network (Figure 36), or via a reference location which realizes the conceptual outfall of the catchment realized by the water body (Figure 37). For example, a fixed landmark, or a point at an associated cross or longitudinal section is considered to realize the outflow of the catchment which is realized by the channel expressed as a flowpath.

Figure 37: Water Body realizing the outfall (UML class diagram, /req/hy_surfacehydro/hydrographicnetwork/waterbody )

#### Hydrographic Network

The HY_HydrographicNetwork feature type specializes the HY_HydroNetwork realization defined in the core model, specifically as aggregate of permanent or temporary bodies of water standing in depressions or moving in channels. If the realized catchment is connected with other catchments via outfall, the hydrographic network is considered connected to the network realizing these catchments. This allows to represent the network, even if logically connected features may or may not be connected at the representation level. If required, an application focused on surface-water bodies contained in channels or depressions may use the defined relationships s to describe the realization of a catchment by the hydrographic network, or network parts associated with the channel network.

HY_HydrographicNetwork inherits from generalization the realizedCatchment, flowpath and waterEdge, as well as catchmentBoundary and catchmentArea associations, and associates a networkWaterBody. A flowline constraint is defined such that whenever the hydrographic network is a network of flowpaths, the network water body is of type HY_Flowpath, and a waters-edge constraint such that the network water body is of type HY_WaterEdge whenever the hydrographic network is a network of water edges.

The networkWaterBody association relates a surface-water body to the hydrographic network. If required, this association shall be used to identify a water body which realizes the logical catchment either separately, or as part of the network.

| Requirements Class | /req/hy_surfacehydro/hydrographicnetwork/hydrographicnetwork |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_HydrographicNetwork |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/hydrographicnetwork.networkwaterbody |

#### Water Body

The HY_WaterBody feature type specializes the general HY_HydroFeature class. The water body as part of the hydrographic network, either standing in a water pool, or flowing as stream in a watercourse, which are parts of the channel network. A water body may be segmented in vertical sections at right angles to the main (average) direction of flow or along its centerline, and horizontal strata. Conceptually, each water body, or a stratum, is understood as a reservoir used for storage, regulation or control of water recourses.

HY_WaterBody inherits from generalization the name property, and the flow line and water edge constraints. It associates the hydrographicNetwork in which it participates, the containing waterpool and watercourse, the vertical streamCrossSection, streamLongitudinalSection, a horizontal stratum, and a reservoir for storage of water. A stratum-edge constraint is defined such that whenever the water body is part of the hydrographic network, the confined stratum is of type HY_WaterEdge. An outflow-at-section and an outflow-at-landmark constraint are defined such that the outflow of the catchment which is realized by the hydrographic network is of type HY_OutfallRealization whenever an outfall is realized at a fixed landmark, a cross or longitudinal section.

The waterpool association relates to a water body the natural or artificial depression wherein water may stand, incl. large interstices in the ground, such as cave, cavern or a group of these. If required, this association shall be used to identify the pool which continuously or periodically contains standing water.

The watercourse association relates to a water body the natural or man-made channel through or along which water may flow, incl. large interstices in the ground, such as cave, cavern or a group of these. If required, this association shall be used to identify the watercourse which continuously or periodically contains moving water.

The upstreamWaterBody and downstreamWaterBody associations relate to a water body another water body immediately up or downstream. If required, these associations shall be used to identify a water body to trace the hydrographic network without knowing the inflow or outflow of the catchment realized by the water body.

The fixedLandmark association relates to the water body permanent reference location. If required, this association shall be used to identify a fixed landmark which realizes the conceptual outfall of the catchment realized by the water body.

The stratum association relates a horizontal stratum. If required, this association shall be used to identify a layer of consistent characteristics, or a storage zone of a reservoir, carrying a permanent reference location which realizes the outfall of the catchment which is realized by the water body.

The streamCrossSection and streamLongitudinalSection associations relate to the water body a vertical section either at right angles to the main (average) direction of flow, or along a centre line. If required, this association shall be used to identify the vertical section carrying a permanent reference location which realizes the outfall of the catchment which is realized by the water body.

The storage association associates to the water body a reservoir storing water as a resource for future use. If required, this association shall be used to describe storage characteristics of the water body participating in the hydrographic network.

| Requirements Class | /req/hy_surfacehydro/hydrographicnetwork/waterbody |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_WaterBody |
| Dependency | /req/hy_hydrofeature/namedFeature/hydrofeature |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/channel |
| Dependency | /req/hy_surfacehydrofeature/channelnetwork/depression |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/hydrographicnetwork |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/stratum |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/crosssection |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/longitudinalsection |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.fixedlandmark |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.waterpool |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.watercourse |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.hydrographicnetwork |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.upstreamwaterbody |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.downstreamwaterbody |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.stratum |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.streamcrosssection |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody.streamlongitudinalsection |

#### Water-Body Stratum

The HY_WaterBodyStratum feature type describes a horizontal layer in a stratified water body determined by differences in thermal or salinity characteristics or by oxygen or nutrient content, or by virtual storage zones of a reservoir. HY_WaterBodyStratum carries the properties: stratumType and benchmark.

The stratumType attribute characterizes in general the stratum using a term from a controlled vocabulary. If required, an implementation may use the Scoped Name type defined in the ISO 19103: Conceptual Schema.

The benchmark association relates to the water body permanent reference location. If required, this association shall be used to identify a benchmark which realizes the conceptual outfall of the catchment realized by the water body.

| Requirements Class | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbodystratum |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_WaterBodyStratum |
| Dependency | /iso/19103/ |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbodystratum.benchmark |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbodystratum.stratumtype |

#### Cross-Section and Longitudinal Section

The HY_CrossSection and HY_LongitudinalSection feature types conceptualize the segmentation of a water body or a containing channel through vertical sections. Taking into account the conceptual separation of a watercourse, the cross section concept refers to both the cross section of a water body orthogonal to the direction of flow, and to the transversal bed profile of a channel.

Both types of vertical section associate permanent reference locations: crossSectionPoint and longitudalinalSectionPoint. If required, this associations shall be used to identify a reference location at a vertical section which realizes the conceptual outfall of the catchment realized by the associated channel or water body.

| Requirements Class | /req/hy_surfacehydrofeature/hydrographicnetwork/crosssection |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_CrossSection |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment/outfallrealization |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/crosssection.crosssectionpoint |

| Requirements Class | /req/hy_surfacehydrofeature/hydrographicnetwork/longitudinalsection |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_LongitudinalSection |
| Dependency | /req/hy_hydrofeature/hydrocomplex/catchment/outfallrealization |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/longitudinalsection.longitudinalsectionpoint |

#### Water-LiquidPhase and Water-SolidPhase

The HY_Water_LiquidPhase and HY_Water_SolidPhase feature types provide simple concepts of the accumulation of water in water bodies. This definition refers to the matter accumulated to a mass of water. In its liquid form water is considered accumulated in water bodies; in its solid phase water may be accumulated after melting, or as a layer of ice or snow on an open water body. The accumulation of water in the atmosphere or below the land surface, e.g. rain, soil moisture or groundwater, is not in scope of this standard, as well as the accumulation of snow and ice in glaciers which is subject of glaciology science. Contextually related information models may use the HY_Water_LiquidPhase and HY_Water_SolidPhase feature types to build relationships to an accumulating water body, and ultimately to the catchment realized either by the water body or by the network of which the water body is part.

HY_Water_LiquidPhase carries the association accumulatingWaterBody; HY_Water_SolidPhase associates two properties: snowmelt and coveredWaterBody. If required, these associations shall be used to identify the water body (part of the network) where liquid water as a material is accumulated.

| Requirements Class | /req/hy_surfacehydrofeature/hydrographicnetwork/waterliquidphase |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_WaterLiquidPhase |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbodystratum |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/waterliquidphase.accumulatingwaterbody |

| Requirements Class | /req/hy_surfacehydrofeature/hydrographicnetwork/watersolidphase |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_WaterSolidPhase |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterliquidphase |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/watersolidphase.coveredwaterbody |
| Requirement | /req/hy_surfacehydrofeature/hydrographicnetwork/watersolidphase.snowmelt |

### The Surface-Water Body types

The Surface Water model defines typical specializations of a water body on the land surface. Being a specialization of the HY_WaterBody class each subtypes inherits the stratum, waterpool, watercourse, upstreamWaterBody, downstreamWaterBody, fixedLandmark, streamCrossSection and streamLongitudinalSection properties. From the general HY_HydroFeature class the special water bodies inherit the name property, which allows to handle names given to them in cross-jurisdiction and multi-lingual contexts. Each specialization is understood to be part of the hydrographic network. In other contexts other specializations, or a typical segmentation may exist, that not conforms to the types defined in this standard.

The HY_River feature type defines the existence of body of surface water, participating in a hydrographic network, special due to its property to permanently or temporarily flow.

The HY_Canal feature type defines the existence of body of surface water, participating in a hydrographic network, special due to its artificial origin (man-made).

The HY_Lake feature type defines the existence of body of surface water, participating in a hydrographic network, special due to its considerable size.

The HY_Impoundment feature type defines the existence of body of surface water, participating in a hydrographic network, special due to be formed by collecting water, as by a dam.

The HY_Lagoon feature type defines the existence of body of surface water, participating in a hydrographic network, special due to its shallow depth and interaction with the open sea.

The HY_Estuary feature type defines the existence of body of surface water, participating in a hydrographic network, special due to branching and its interaction with the open sea.

### The Storage model

The Storage model (Figure 38) provides a concept to describe any water body, in terms of a reservoir storing water for future use. The separate storage model allows to describe the hydrographic network without the details of storage capacities that a water body may have, and vice versa storage reservoirs to be referenced independent of their role within the hydrographic network.

Figure 38: Reservoir realizing an outfall (UML class diagram, /req/hy_surfacehydrofeature/storage/reservoir )

The HY_Reservoir feature type describes the water body, either natural or man-made, used for storage, regulation and control of water resources. The reservoir concept refers to a volume of water managed in zones between operating levels. HY_Reservoir associates to a reservoir the storedWaterBody and a reservoirZone.

| Requirements Class | /req/hy_surfacehydrofeature/storage/reservoir |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_Reservoir |
| Dependency | /req/hy_surfacehydrofeature/hydrographicnetwork/waterbody |
| Dependency | /req/hy_surfacehydrofeature/hydrocomplex/outfallrealization |
| Requirement | /req/hy_surfacehydrofeature/storage/reservoir.storedwaterbody |
| Requirement | /req/hy_surfacehydrofeature/storage/reservoir.reservoirzone |

## The Hydrometric Network application schema

The Hydrometric Network application schema (Figure 40) defines a logical model to take into account a network of hydrometric stations as a specific realization of the catchment in the perspective of hydrologic observation, without the detail of an observation strategy. The Hydrometric Network specifies the core concepts defined in the abstract Hydro Feature application schema (Figure 39). The general concept is that of a network of logically connected hydrometric stations realizing as a whole the catchment. This enables contextually related information models to relate monitoring stations and observing posts to hydrologic features, finally to the realized catchment, as usually required in the context of environmental reporting or when interpreting, comprising and processing observation results.

Figure 39: Hydrometric Network – dependencies

The hydrometric network model introduces the concept of a 'position on river' which allows an hydrologic station, even free from position, to be the realization of the logical outfall. This supports to establish upstream-downstream relationships between hydrometric features, to assign a position relative to a 'fixed' outfall, or to place a feature of interest relative to the hydrometric station.

Figure 40: Hydrometric network model (UML class diagram, /req/hy_hydrometricfeature/hydrometricnetwork )

The HY_HydrometricNetwork feature type specializes the HY_HydroNetwork realization defined in the core model specifically as an aggregate of hydrometric features. HY_HydrometricNetwork inherits from generalization the realizedCatchment, flowpath and waterEdge, as well as catchmentBoundary and catchmentArea associations. The networkStation association relates a monitoring station at which data on water are obtained. An outflow-at-station constraint is defined such that the outflow of the catchment which is realized by the channel network is of type HY_OutfallRealization whenever an outfall is realized through the hydrometric feature.

If required, this association shall be used to identify the hydrometric station, which realizes the logical catchment either separately, or as part of the hydrometric network.

The HY_HydrometricFeature associates the hydrometricNetwork in which it participates and a positionOnRiver. The positionOnRiver association relates the hydrometric feature to a permanent reference location. If required, this association shall be used to identify a reference location which realizes the conceptual outfall of the catchment corresponding to the hydrometric feature.

| Requirements Class | /req/hy_hydrometricfeature/hydrometricnetwork |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_HydrometricNetwork |
| Dependency | /req/hy_hydrofeature/hydrocomplex/hydronetwork |
| Dependency | /req/hy_hydrometricfeature/hydrometricfeature |
| Requirement | /req/hy_hydrometricfeature/hydrometricnetwork.networkstation |

| Requirements Class | /req/hy_hydrometricnetwork/hydrometricfeature |
| --- | --- |
| Target type | Implementation Schema |
| Name | HY_HydrometricFeature |
| Dependency | /req/hy_hydrofeature/hydrocomplex/outfallrealization |
| Dependency | /req/hy_hydrometricfeature/hydrometricnetwork |
| Requirement | /req/hy_hydrometricnetwork/hydrometricfeature.positiononriver |
| Requirement | /req/hy_hydrometricnetwork/hydrometricfeature.hydrometricnetwork |

# ANNEX A Conformance Class Abstract Test Suite (Normative)

## A.1 Introduction

These test suites ascertain the compliance of the conformance targets for the HY_Features specification with the specification itself. Each instance of hydrologic feature data is encoded according to a specific interchange schema, so the role of conformance with the abstract specification is that such an implementation schema can be related to the common definitions of HY_Features.

An implementation schema conforming to HY_Features SHALL provide a formal mapping from one or more Feature Types present in the implementation schema to Feature Types defined in this standard specification, including all mandatory properties defined by the realized HY_Features concept. Default values to be assumed must be specified in this mapping.

## A.2 Conformance class: HY_HydroFeature application schema equivalence

| Conformance Class | /spec/hydrology/hydrofeature/1.0/conf/hy_ hydrofeature |
| --- | --- |
| Test | /conf/uml_hydrofeature/namedfeature/* |
| Test | /conf/uml_hydrofeature/hydrocomplex/* |
| Test | /conf/uml_hydrofeature/positioning/* |
| Requirement | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test purpose | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test method | Inspect the mapping between the data exchange schema and the HY_Features model to determine that all relevant schema elements are mapped to HY_Features equivalents. |
| Test type | Capability |

## A.3 Conformance class: HY_SurfaceHydroFeature application schema equivalence

| Conformance Class | / spec/hydrology/surfacehydrofeature/1.0/conf/hy_ surfacehydrofeature |
| --- | --- |
| Test | /conf/uml_surfacehydrofeature/channelnetwork/* |
| Test | /conf/uml_surfacehydrofeature/hydrographicnetwork/* |
| Test | /conf/uml_surfacehydrofeature/surfacewaterbodies/* |
| Test | /conf/uml_surfacehydrofeature/storage/* |
| Requirement | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test purpose | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test method | Inspect the mapping between the data exchange schema and the HY_Features model to determine that all relevant schema elements are mapped to HY_Features equivalents. |
| Test type | Capability |

## A.4 Conformance class: HY_HydrometricFeature application schema equivalence

| Conformance Class | / spec/hydrology/hydrometricfeature/1.0/conf/hy_ hydrometricfeature |
| --- | --- |
| Test | /conf/uml_ hydrometricfeature/hydrometricfeature/* |
| Requirement | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test purpose | All relevant elements of a data exchange schema including hydrologic features are mapped to equivalent HY_Features elements. |
| Test method | Inspect the mapping between the data exchange schema and the HY_Features model to determine that all relevant schema elements are mapped to HY_Features equivalents. |
| Test type | Capability |

# ANNEX B - Code lists for the HY_Features model

## B.1 Terms identifying a fixed landmark determined to realize the conceptual outfall

| Code list | Realized Outfall Type |
| --- | --- |
| barrage | barrier across a stream provided with a series of gates or other control mechanisms to control the water-surface level upstream, to regulate the flow or to divert water supplies into a canal. |
| bifurcation | division of a stream into two branches. |
| confluence | joining, or the place of junction, of two or more streams. |
| dam | barrier constructed across a valley for impounding water or creating a reservoir. |
| diversion of water | transfer of water from one watercourse to another, such watercourses being either natural or man-made. |
| fork | (1) place where two or more streams flow together to form a larger stream. (2) place where a stream divides into two or more streams. |
| hydrometric station | station at which data on water in rivers, lakes or reservoirs are obtained on one or more of the following elements: stage, streamflow, sediment transport and deposition, water temperature and other physical properties of water, characteristics of ice cover and chemical properties of water. |
| inlet | structure admitting water supplies from the source or through an intake structure built upstream. |
| intake | structure or site, the purpose of which is to control, regulate, divert, and admit water directly from the source, through an inlet built upstream. |
| outlet | opening through which water flows out or is extracted from a reservoir or stream. |
| ponor | hole or opening in the bottom or side of a depression where a surface stream or lake flows either partially or completely underground into a karst groundwater system |
| rapids | reach of a stream where the flow is very swift and shooting, and where the surface is usually broken by obstructions, but has no actual waterfall or cascade. |
| reference climatological station | climatological station the data of which are intended for the purpose of determining climatic trends. This requires long periods (not less than thirty years) of homogeneous records, where man-made environmental changes have been and/or are expected to remain at a minimum. Ideally the records should be of sufficient length to enable the identification of secular changes of climate. |
| river mouth | place of discharge of a river into a sea or a lake. |
| sinkhole | place where water disappears underground in a limestone region. It generally implies water loss in a closed depression or blind valley. |
| source | origin of river. |
| spring | place where water flows naturally from a rock or soil onto land or into a body of surface water. |
| waterfall | vertical fall or the very steep descent of a stream of water. |
| weir | overflow structure which may be used for controlling upstream water level or for measuring discharge or for both. |

## B.2 Terms commonly used in hydrology to describe a spatial relation between two points

| Code list | Relative Position Type |
| --- | --- |
| at | located at the (reference) point |
| between | located between two (reference) points |
| downstream | located downstream of the (reference) point, e.g. in the direction of the current in a river or stream. |
| left | located left-hand of the (reference) point when facing downstream. |
| nearby | located in a short distance to the (reference) point. |
| right | located right-hand of the (reference) point when facing downstream. |
| upstream | located upstream of the (reference) point, e.g. in the direction towards the source of a stream. |

## B.3 Terms commonly used in hydrology to describe a drainage pattern

| Code list | Drainage Pattern |
| --- | --- |
| annular | main rivers have circular pattern with subsidiary channels at right angles. |
| centripetal | streams flow inward to center. |
| dendritic | spreading treelike arrangement; no evident orientation of channels (random orientation). |
| distributary | one main channel divides into many channel-ways ending with many outlets. |
| parallel | main channels regularly spaced and parallel or sub-parallel to each other; tributaries join at very acute angles. |
| pinnate | featherlike, closely grouped , short tributaries (fine texture). |
| radial | streams flow outward from center. |
| rectangular | drainage forms a perpendicular net with the two directions equally developed. |
| trellis | a dominant drainage direction with a secondary direction perpendicular to this; primary tributaries join main stream at right angles, secondary tributaries parallel main stem. |

## B.4 Terms commonly used to indicate the type of name usage.

| Codelist | Usage Type |
| --- | --- |
| conventional | accepted, used, or practiced by most people ('agreed by convention') |
| historical | restricted to or based on fact in history |
| official | ordered or allowed by those in authority |
| vernacular | used in or suitable for speech, usually not used in formal writing |

# ANNEX C:  HY_Features  - AHGF Mapping

This is a descriptive mapping for the Australian Hydrological Geospatial Fabric (AHGF) [11]. It is intended to provide an understanding of the basic relationship of HY_Features concepts and the AHGF hydrologic feature implementation.

## C.1 Catchment Model

| HY_Features Name | AHGF Class | Comment |
| --- | --- | --- |
| HY_Catchment | ConCatID | Logical features identified via ConCatID with topological relationships defined via ConNodeID (HY_Outfall). Can also be seen as the contributing set of logical features sharing a common ConCatID. |
| HY_DendriticCatchment | ConCatID | All logical HY_Catchment features are constrained to be dendritic in the Geofabric. |
| HY_InteriorCatchment | Not represented | All catchment areas defined by a particular DEM are considered to flow to a single common outlet. Note: Areas not flowing to a common outlet are aggregated with areas that do according to saddle points in the terrain model. |
| HY_CatchmentAggregate | Collections of ConNodeID | Network following connectivity via ConNodeID encapsulating divergent flows to remain dendritic in nature. |
| HY_Outfall | ConNodeID | All ConNodeID act in the role of outflow node (To_Node) for one or more ConCatID catchment(s) and most also act in the role of inflow node (From_Node) for a single ConCatID catchment. |
| HY_Outfallrealization | AHGFNetworkNode | HY_ReferenceLocation can be used as a reference location for any point associated with a feature. In reality this will normally be a realization of a ConNodeID (with associated coordinates) or the Geofabric realization of event locations (AHGFNetworkNode::GhostNode) used to register Hydrometric features on the network. |
| HY_CatchmentRealization | Feature identified by ConCatID | Any feature or collection of features identified by a single ConCatID. Single instances of AHGFContractedCatchment and AHGFLink as well as collections of AHGFNetworkStream, AHGFNetworkNode, AHGFCatchment (low level sub-catchments) and AHGFWaterbody. Note: features of subtype AHGFContractedCatchment::NoFlowArea do not have a ConNodeID (i.e. No HY_Outfall) |
| HY_CatchmentArea | AHGF Contracted Catchment, AHGFCatchment | A single AHGFContractedCatchment or a collection of AHGFCatchment (both 2D simple surfaces derived from a common Digital Elevation model). |
| HY_CatchmentBoundary | Not represented | Can be seen as the boundary (or boundaries) of the polygon feature(s) that realise HY_CatchmentArea. |
| HY_CartographicRealization | AHGFMappedStream | AHGFMappedStream is a feature class of the Geofabric Surface Cartography product, perhaps all feature classes in this product can be considered as HY_CartographicRealization. |

## C.2 Hydrographic Network

| HY_Features Name | AHGF Class | Comment |
| --- | --- | --- |
| HY_Hydrographic Network | AHGFNetworkStream | Subset of AHGFNetworkStream features (subtypes: NetworkFlowSegment & NetworkWaterAreaSegment) provides a particular HY_CatchmentRealization. This network realization can be supplemented with its corresponding HY_ChannelNetwork instance. |
| HY_WaterBody | AHGFNetworkStream | AHGFNetworkStream features (subtypes: NetworkFlowSegment & NetworkWaterAreaSegment) |
| HY_ChannelNetwork | AHGFNetworkStream | Subset of AHGFNetworkStream:: NetworkArtificialFlowSegment provides a particular HY_CatchmentRealization. This network realization is only complete when combined with the corresponding HY_HydrographicNetwork instance. |
| HY_Depression | Not represented |  |
| HY_Channel | AHGFNetworkStream | AHGFNetworkStream features (subtype: NetworkArtificialFlowSegment). For version 3 products AHGFNetworkStream features are related to the relevant logical HY_Catchment via ConCatID. |
| HY_Reservoir | AHGFWaterbody::Reservoir | Reservoir subtype of AHGFWaterbody. |
| HY_FlowPath | AHGFLink | Note: Geofabric currently has no FlowPath for headwater (AHGFContractedCatchment) areas. |
| HY_WaterEdge | Not represented |  |
| HY_LongitudinalSection | Not represented |  |
| HY_CrossSection | Not represented |  |
| HY_WaterBodyStratum | Not represented |  |
| HY_Water_LiquidPhase | Not represented |  |
| HY_Water_SolidPhase | Not represented |  |

## C.3 Hydrometric Network

| HY_Features Name | AHGF Class | Comment |
| --- | --- | --- |
| HY_HydrometricNetwork | Not represented | Possible future plans to produce a geoschematic representation of the hydrometric features at some stage. |
| HY_HydrometricFeature | AHGFNetworkNode:: AHGFGhostNode | Ghost Nodes are a representation of Hydrometric features located on a Hydrographic Network representation. Note: Currently, V3 has a separate AHGFGhostNode feature class for ghost nodes. There are plans to include these features as part of the Hydrology Reporting Catchments product forming outfalls for AHGFContractedCatchment (HY_DendriticCatchment) features. |
| HY_RiverReferenceSystem | Not represented | Measures are not yet explicitly included in AHGF. |
| HY_IndirectPostition | Not represented | Measures are not yet explicitly included in AHGF. |

# ANNEX D:  HY_Features  - NHDPlus  Mapping

This is a descriptive mapping for the USGS National Hydrography Dataset Plus (NHDPlus)[10]. It is intended to provide an understanding of the basic relationship of HY_Features concepts and the NHDPlus hydrologic feature implementation.

## D.1 Catchment Model

| HY_Features Name | NHDPlus Name | Comment |
| --- | --- | --- |
| HY_Catchment | comid catchment | The comid catchment is the feature that takes part in PlusFlow topology table, has associated accumulated characteristics, etc. Could also be the collection of catchments that contribute to a given reachcode reach, HUC watershed outlet, or other identifiable watershed outlet. |
| HY_DendriticCatchment | comid catchment | NHDPlus catchments not following any diversions in the flow tables |
| HY_InteriorCatchment | comid catchment | NHDPlus catchments that do not contribute flow. |
| HY_CatchmentAggregate | collection of comid catchments | Including interior catchments and not following diversions in the flow tables |
| HY_Outfall | fromNode or toNode | NHDPlus nodes are inflow (fromNode) and outflow (toNode) nodes of a given comid catchment. |
| HY_OutfallRealization | fromNode or toNode location, point location of point events, etc. | HY_ReferenceLocation can be used as a reference location for any point associated with a feature. Typically this is for outfalls and monitoring locations, but may be used for many other feature types. |
| HY_CatchmentRealization | Any entity that is identified by a comid | In NHDPlus, the comid identifier represents an unrealized catchment. Any geometric or topologic data that is referenced to a comid can be said to realize that catchment. This includes upstream aggregations of the network or catchment areas to form complete watersheds. |
| HY_CatchmentArea | Not Represented | While the polygon representing a catchment might be thought of as an area, the subset of a DEM or another land cover dataset would be more in line with the meaning of CatchmentArea. |
| HY_CatchmentBoundary | comid catchment polygon | The polygon representing a comid catchment should be thought of as both the catchmentBoundary |
| HY_CartographicRealization | A map of a catchment | The NHDPlus dataset doesn't include any, but if a map view of a catchment is created at any scale, it could be said to be a cartographic visualization realization of the catchment. |

## D.2 Hydrographic Network Model

| HY_Features Name | NHDPlus Name | Comment |
| --- | --- | --- |
| HY_HydrographicNetwork | collection of flowlines and waterbodies | The collection of perennial and ephemeral flowlines as well as so-called double line streams and on-network lakes within any collection of catchments (an HY_Catchment) can be considered it's hydrographic network. |
| HY_WaterBody | Perennial flowlines and waterbodies | Perennial flowlines are thought to represent water bodies as well as waterbody polygons that represent wide streams and lakes. These features indicate that there is water contained in some channel or other container. |
| HY_ChannelNetwork | Not represented |  |
| HY_Depression | Not represented |  |
| HY_Channel | Ephemeral flowlines. | While NHD doesn't have an explicit channel concept, ephemeral flowlines can be thought of as a channel in that they indicate that water can flow there, but may not be present in the flowline container at all times. |
| HY_Reservoir | waterbodies that are reservoirs | Any waterbody that can be categorized as a reservoir |
| HY_FlowPath | flowlines and reaches | A flowline or reach is the linear representation of a catchment. For NHD Events, the reach is the flowpath because it is the linear element used for linear referencing. |
| HY_WaterEdge | Not represented |  |
| HY_LinearSegment | flowline | For reachcode linear referencing, the reachcode flowpath is made up of a collection of linear segments that are flowlines |
| HY_LongitudinalSection | Not represented |  |
| HY_CrossSection | Not represented |  |
| HY_WaterBodyStratum | Not represented |  |
| HY_Water_LiquidPhase | Not represented |  |
| HY_Water_SolidPhase | Not represented |  |

## D.3 Hydrometric Network Model

| HY_Features Name | NHDPlus Name | Comment |
| --- | --- | --- |
| HY_HydrometricNetwork | Not Represented |  |
| HY_HydrometricFeature | event | A hydrometric feature, such as a stream gaging station, is represented as an event. |
| HY_RiverReferenceSystem | reachcode and measure | A reachcode reach is the river reference system's shape, the origin is the outlet of the reach, and the indirect position is the measure. |
| HY_IndirectPosition | measure | The measure is an indirect position of type relative position because it is a percent. |

# ANNEX E:  HY_Features  - INSPIRE Hydrography Theme

This table assigns core features as defined in the INSPIRE Hydrography theme [2] to HY_Features core concepts. It also indicates relationship between both terms and potential conditions in which they can be associated.

This table should not be understood in terms of a  ‘conceptual mapping’ of clearly defined relationships between two logical concepts, but rather as a simple approach to show compatibility of concepts, based on the meaning expressed in the definitions, and without imposing a particular implementation. Since each dataset that implements the INSPIRE theme will have a different legacy of documentation and information modeling, a conceptual mapping would be expressed differently.  Applications implementing the INSPIRE Hydrography theme can use this listing to understand how a particular dataset may relate to the HY_Features concepts.

## E.1 Catchment Model

| HY_Features Name | Relationship between concepts | INSPIRE Hydrography Name + potential condition(s) | Definition / Description |
| --- | --- | --- | --- |
| HY_Catchment | Is like | DrainageBasin In case that HY_Catchment is realized by HY_CatchmentArea | Area having a common outlet for its surface runoff. NOTE 1 Regarding the different classifications of drainage basins, no distinction is made between drainage basins / sub-basins since this will vary with application. It is possible to build basins from other basins. NOTE 2 The outlet of a drainage basin may be a canal or a lake. NOTE 3 Synonyms for drainage basin include: catchment; catchment area; drainage area; river basin; watershed. |
| HY_DendriticCatchment | Is narrower than | DrainageBasin In case that HY_Catchment is realized by HY_CatchmentArea With one outlet | Area having a common outlet for its surface runoff. NOTE 1 Regarding the different classifications of drainage basins, no distinction is made between drainage basins / sub-basins since this will vary with application. It is possible to build basins from other basins. NOTE 2 The outlet of a drainage basin may be a canal or a lake. NOTE 3 Synonyms for drainage basin include: catchment; catchment area; drainage area; river basin; watershed. |
| HY_InteriorCatchment |  | Not represented |  |
| HY_CatchmentAggregate | Is like | RiverBasin In case that HY_Catchment is realized by HY_CatchmentArea | The area of land from which all surface run-off flows through a sequence of streams, rivers and, possibly, lakes into the sea at a single river mouth, estuary or delta. |
| HY_Outfall | Is like | HydroNode In case that HY_Catchment is realized by HY_FlowPath | A node within the hydrographic network. NOTE May represent a physical confluence, bifurcation/confluence/vanishing point etc., or it may be associated with a hydrographic point of interest or facility. |
| HY_OutfallRealization | Is like | HydroPOI | A natural place where water appears, disappears or changes its flow. EXAMPLE Fluvial points (waterfall, cascade, rapids, breaker), spring/water hole (spring, source, geyser, thermal spring, natural fountain, well, also fumarole, artesian), sinkhole (sinkhole, drainage loss). NOTE A hydro point of interest may create a flow constriction in the network. |
| HY_CatchmentRealization |  | Not represented |  |
| HY_CatchmentArea | Is like | DrainageBasin | Area having a common outlet for its surface runoff. NOTE 1 Regarding the different classifications of drainage basins, no distinction is made between drainage basins / sub-basins since this will vary with application. It is possible to build basins from other basins. NOTE 2 The outlet of a drainage basin may be a canal or a lake. NOTE 3 Synonyms for drainage basin include: catchment; catchment area; drainage area; river basin; watershed. |
| HY_CatchmentBoundary |  | Not represented |  |
| HY_CartographicRealization |  | Not represented |  |

## E.2 Hydrographic Network Model

| HY_Features Name | Relationship between concepts | INSPIRE Hydrography Name + potential condition(s) | Definition / Description |
| --- | --- | --- | --- |
| HY_HydrographicNetwork | Is narrower than | WatercourseLinkSequence In case that features of interest are water bodies and not channels. AND In case that network is non branching. | A sequence of watercourse links representing a non-branching path through a hydrographic network. |
| HY_WaterBody | Is narrower than | WatercourseLink In case that features of interest are water bodies and not channels. | A segment of a watercourse within a hydrographic network. A watercourse link may be fictitious, with no direct correspondence to a real-world object and included only to ensure a closed network. |
| HY_ChannelNetwork | Is narrower than | WatercourseLinkSequence In case that features of interest are channels and not water bodies. AND In case that network is non branching. | A sequence of watercourse links representing a non-branching path through a hydrographic network. |
| HY_Depression | Is like | StandingWater In case that geometry is surface or point. | Any known inland waterway body. EXAMPLE Lake/pond, reservoir, river/stream, etc. NOTE: May include islands, represented as 'holes' in its geometry. Islands may be surrounded by a shore and / or land-ware boundary. |
| HY_Channel | Is narrower than | WatercourseLink In case that features of interest are channels and not water bodies. | A segment of a watercourse within a hydrographic network. A watercourse link may be fictitious, with no direct correspondence to a real-world object and included only to ensure a closed network. |
| HY_Reservoir | Is like | StandingWater In case that geometry is surface or point. | Any known inland waterway body. EXAMPLE Lake/pond, reservoir, river/stream, etc. NOTE: May include islands, represented as 'holes' in its geometry. Islands may be surrounded by a shore and / or land-ware boundary. |
| HY_FlowPath | Is like | WatercourseLink With a fictitious representation | A segment of a watercourse within a hydrographic network. A watercourse link may be fictitious, with no direct correspondence to a real-world object and included only to ensure a closed network. |
| HY_WaterEdge | Is like | LandWaterBoundary | The line where a land mass is in contact with a body of water. |
| HY_LongitudinalSection |  | Not represented |  |
| HY_CrossSection |  | Not represented |  |
| HY_WaterBodyStratum |  | Not represented |  |
| HY_Water_LiquidPhase |  | Not represented |  |
| HY_Water_SolidPhase |  | Not represented |  |

# ANNEX F:  HY_Features  - SANDRE  Mapping

This is a descriptive mapping for the French National Service for Water Data and Reference-dataset Management (SANDRE) [12]. It is intended to provide an understanding of the basic relationship of HY_Features concepts and the SANDRE hydrologic feature dictionary for Hydrography.

Please notice that terms for SANDRE are based on French words.

## F.1 Catchment Model

| HY_Features Name | SANDRE Name | Comment |
| --- | --- | --- |
| HY_Catchment | RegionHydro OR SecteurHydro OR SousSecteurHydro OR ZoneHydro | SANDRE make distinction between basins and sub-basins |
| HY_DendriticCatchment | RegionHydro OR SecteurHydro OR SousSecteurHydro OR ZoneHydro | SANDRE: all catchments are dendritic |
| HY_InteriorCatchment | Not represented |  |
| HY_CatchmentAggregate | RegionHydro OR SecteurHydro OR SousSecteurHydro | SANDRE: RegionHydro are aggregation of SecteurHydro that are aggregation of SousSecteurHydro that are aggregation of ZoneHydro |
| HY_Outfall | NoeudHydrographique | HY_Outfall can be seen as a specific NoeudHydrographique |
| HY_OutfallRealization | NoeudHydrographique | HY_Outfall can be seen as a specific NoeudHydrographique |
| HY_CatchmentRealization | Not represented |  |
| HY_CatchmentArea | RegionHydro OR SecteurHydro OR SousSecteurHydro OR ZoneHydro | While the polygon representing a catchment might be thought of as an area, the subset of a DEM or another land cover dataset would be more in line with the meaning of CatchmentArea. |
| HY_CatchmentBoundary | LimiteHydroBassin | SANDRE : LimiteHydroBassin is the boundary of a catchment. It has a linear representation. |
| HY_CartographicRealization | Not represented |  |

## F.2 Hydrographic Network Model

| HY_Features Name | SANDRE Name | Comment |
| --- | --- | --- |
| HY_HydrographicNetwork | Not represented | SANDRE focuses on channels, not on water bodies |
| HY_WaterBody | Not represented | SANDRE focuses on channels, not on water bodies |
| HY_ChannelNetwork | CoursEau OR TronconHydrographique | SANDRE: CoursEau is an aggregation of TronconHydrographique that is an aggregation of TronconHydrograElt. All of them have a linear representation. |
| HY_Depression | EntiteHydroSurface OR EltHydroSurface | SANDRE: EntiteHydroSurface is an aggregation of EltHydroSurface. Both have a surfacic representation. |
| HY_Channel | TronconHydrographique OR TronconHydrograElt OR EltHydroSurface | SANDRE: CoursEau is aggregation of TronconHydrographique that is an aggregation of TronconHydrograElt. All of them have a linear representation. EntiteHydroSurface |
| HY_Reservoir | EntiteHydroSurface OR EltHydroSurface OR PointEauIsole | SANDRE: EntiteHydroSurface is an aggregation of EltHydroSurface. Both have a surfacic representation. PointEauIsole can be used in case that HY_Reservoir is not linked to the hydrographic network and is represented by a point |
| HY_FlowPath | Not represented |  |
| HY_WaterEdge | Not represented |  |
| HY_LongitudinalSection | Not represented |  |
| HY_CrossSection | Not represented |  |
| HY_WaterBodyStratum | Not represented |  |
| HY_Water_LiquidPhase | Not represented |  |
| HY_Water_SolidPhase | Not represented |  |

## F.3 Hydrometric Network Model

| HY_Features Name | NHDPlus Name | Comment |
| --- | --- | --- |
| HY_HydrometricNetwork | Not Represented |  |
| HY_HydrometricFeature | Not Represented |  |
| HY_RiverReferenceSystem | Not Represented |  |
| HY_IndirectPosition | Not Represented |  |

# ANNEX G: Bibliography

- European Parliament and the Council: Directive 2000/60/EC of the European Parliament and of the Council of 23 October 2000 establishing a framework for the Community action in the field of water policy (EU Water Framework Directive). European Commission (2000)

- INSPIRE Thematic Working Group Hydrography: D2.8.I.8 Data Specification on Hydrography – Technical Guidelines. D2.8.I.8_v3.1. European Commission (2014), http://inspire.ec.europa.eu/documents/Data_Specifications/INSPIRE_DataSpecification_HY_v3.1.pdf

- OGC Hydrology Domain Working Group Charter. OGC 08-095r5, Open Geospatial Consortium (2008), https://portal.opengeospatial.org/files/?artifact_id=59773

- OGC WaterML 2.0: Part 1- Timeseries. OGC 10-126r4, Open Geospatial Consortium (2012). https://portal.opengeospatial.org/files/?artifact_id=57222

- OGC HY_Features: a Common Hydrologic Feature Model: Discussion Paper. OGC 11-039r3, Open Geospatial Consortium (2013), https://portal.opengeospatial.org/files/?artifact_id=55157

- OGC WaterML 2.0: Part 4- GroundwaterML 2 (GWML2). OGC 16-032r2, Open Geospatial Consortium (2016)

- Hydrologic units, hydrologic unit codes, and hydrologic unit names (Modified from Slack and Landwehr, 1992 and Seaber, Kapinos, & Knapp, 1987). US Geological Survey (1992), https://water.usgs.gov/nawqa/sparrow/wrr97/geograp/hucs.txt

- Guide to Hydrological Practices. Volume I: Hydrology – From Measurement to Hydrological Information. World Meteorological Organization (ed.). WMO (Series), no168. WMO , Geneva  (2012), http://www.whycos.org/chy/guide/168_Vol_I_en.pdf.

- International Glossary of Hydrology / Glossaire International d'Hydrologie. World Meteorological Organization, United Nations Educational, Scientific and Cultural Organization (eds.). WMO (Series), no385. WMO, Geneva (2016). ISBN 978-92-63-03385-8. ISBN 978-92-3-001154-3.

- McKay, L., Bondelid, T., Dewald, T., Johnston, J., Moore, R., and Rea, A.: NHDPlus Version 2: User Guide, Horizon Systems (2012), ftp://ftp.horizon-systems.com/nhdplus/NHDPlusV21/Documentation/NHDPlusV2_User_Guide.pdf

- Bureau of Meteorology: Australian Hydrological Geospatial Fabric (Geofabric) version 3.0, Product Guide, Bureau of Meteorology, Australia (2015), http://www.bom.gov.au/water/geofabric/documents/v3_0/ahgf_productguide_V3_0_release.pdf

- National service for water-data and reference-dataset management (2010), http://www.sandre.eaufrance.fr/concept/national-service-water-data-and-reference-dataset-management?lang=en

# ANNEX H: Revision history

| Date | Release | Author | Paragraph modified | Description |
| --- | --- | --- | --- | --- |
| 08/10/2014 | 0.1 | Irina Dornblut | Initial version | Initial draft in OGC template |
| 12.11.2014 | 0.2 | Irina Dornblut | Section7, Annex D included | Editing requirements classes, Annex D included |
| 15.08.2016 | 0.3 | David Blodgett; Irina Dornblut | Completely revised version | Draft in OGC template |
| 18.08.2016 | 0.4 | Irina Dornblut | Figs7,8,9,10,11,13,14 | added |
| 18.31.2016 | 0.5 | David Blodgett, Darren Smith, David Arctur | Entire Document | General edit for clarity and consistency. |
| 12.09.2016 | 0.6 | Irina Dornblut | Entire document | edits for clarity and consistency. All Figures updated Translation into American |

https://cite.opengeospatial.org/

also online, http://webworld.unesco.org/water/ihp/db/glossary/glu/aglo.htm